| Class or division | Subsidiary hazard | UN No. | Proper shipping name |
| --- | --- | --- | --- |
| 1 |  | 0190 | CLASS 1 SAMPLES, EXPLOSIVE, other than initiating explosive |
| 1.1A 1.1B 1.1C 1.1C 1.1C 1.1C 1.1D 1.1D 1.1E 1.1F 1.1G 1.1L 1.1L |  | 0473 0461 0462 0474 0497 0498 0463 0475 0464 0465 0476 0354 0357 | Division 1.1 SUBSTANCES, EXPLOSIVE, N.O.S. COMPONENTS, EXPLOSIVE TRAIN, N.O.S. ARTICLES, EXPLOSIVE, N.O.S. SUBSTANCES, EXPLOSIVE, N.O.S. PROPELLANT, LIQUID PROPELLANT, SOLID ARTICLES, EXPLOSIVE, N.O.S. SUBSTANCES, EXPLOSIVE, N.O.S. ARTICLES, EXPLOSIVE, N.O.S. ARTICLES, EXPLOSIVE, N.O.S. SUBSTANCES, EXPLOSIVE, N.O.S. ARTICLES, EXPLOSIVE, N.O.S. SUBSTANCES, EXPLOSIVE, N.O.S. |
|  |  |  | Division 1.2 |
| 1.2B |  | 0382 | COMPONENTS, EXPLOSIVE TRAIN, N.O.S. |
| 1.2C |  | 0466 | ARTICLES, EXPLOSIVE, N.O.S. |
| 1.2D |  | 0467 | ARTICLES, EXPLOSIVE, N.O.S. |
| 1.2E |  | 0468 | ARTICLES, EXPLOSIVE, N.O.S. |
| 1.2F |  | 0469 | ARTICLES, EXPLOSIVE, N.O.S. |
| 1.2K | 6.1 | 0020 | AMMUNITION, TOXIC with burster, expelling charge or propelling charge |
| 1.2L | 4.3 | 0248 | CONTRIVANCES, WATER-ACTIVATED with burster, expelling charge or propelling charge |
| 1.2L |  | 0355 | ARTICLES, EXPLOSIVE, N.O.S. |
| 1.2L |  | 0358 | SUBSTANCES, EXPLOSIVE, N.O.S. |
|  |  |  | Division 1.3 |
| 1.3C |  | 0132 | DEFLAGRATING METAL SALTS OF AROMATIC NITRO-DERIVATIVES, N.O.S. |
| 1.3C |  | 0470 | ARTICLES, EXPLOSIVE, N.O.S. |
| 1.3C |  | 0477 | SUBSTANCES, EXPLOSIVE, N.O.S. |
| 1.3C |  | 0495 | PROPELLANT, LIQUID |
| 1.3C |  | 0499 | PROPELLANT, SOLID |
| 1.3G |  | 0478 | SUBSTANCES, EXPLOSIVE, N.O.S. |
| 1.3K | 6.1 | 0021 | AMMUNITION, TOXIC with burster, expelling charge or propelling charge |
| 1.3L | 4.3 | 0249 | CONTRIVANCES, WATER-ACTIVATED with burster, expelling charge or propelling charge |
| 1.3L |  | 0356 | ARTICLES, EXPLOSIVE, N.O.S. |
| 1.3L |  | 0359 | SUBSTANCES, EXPLOSIVE, N.O.S. |
| 1.4B 1.4B 1.4C 1.4C 1.4C 1.4D 1.4D 1.4E 1.4F 1.4G 1.4G 1.4S 1.4S 1.4S |  | 0350 0383 0351 0479 0501 0352 0480 0471 0472 0353 0485 0349 0384 0481 | Division 1.4 ARTICLES, EXPLOSIVE, N.O.S. COMPONENTS, EXPLOSIVE TRAIN, N.O.S. ARTICLES, EXPLOSIVE, N.O.S. SUBSTANCES, EXPLOSIVE, N.O.S. PROPELLANT, SOLID ARTICLES, EXPLOSIVE, N.O.S. SUBSTANCES, EXPLOSIVE, N.O.S. ARTICLES, EXPLOSIVE, N.O.S. ARTICLES, EXPLOSIVE, N.O.S. ARTICLES, EXPLOSIVE, N.O.S. SUBSTANCES, EXPLOSIVE, N.O.S. ARTICLES, EXPLOSIVE, N.O.S. COMPONENTS, EXPLOSIVE TRAIN, N.O.S. SUBSTANCES, EXPLOSIVE, N.O.S. |
| 1.5D |  | 0482 | Division 1.5 SUBSTANCES, EXPLOSIVE, VERY INSENSITIVE (SUBSTANCES, EVI), N.O.S. |
| 1.6N |  | 0486 | Division 1.6 ARTICLES, EXPLOSIVE, EXTREMELY INSENSITIVE (ARTICLES, EEI) |

APPENDIX A

| Class or division | Subsidiary hazard | UN No. | Proper shipping name |
| --- | --- | --- | --- |
|  |  |  | CLASS 2 |
|  |  |  | Class 2.1 |
|  |  |  | Specific entries |
| 2.1 |  | 1964 | HYDROCARBON GAS MIXTURE, COMPRESSED, N.O.S. |
| 2.1 |  | 1965 | HYDROCARBON GAS MIXTURE, LIQUEFIED, N.O.S. |
| 2.1 |  | 3354 | INSECTICIDE GAS, FLAMMABLE, N.O.S. |
|  |  |  | General entries |
| 2.1 |  | 1954 | COMPRESSED GAS, FLAMMABLE, N.O.S. |
| 2.1 |  | 3161 | LIQUEFIED GAS, FLAMMABLE, N.O.S. |
| 2.1 |  | 3167 | GAS SAMPLE, NON-PRESSURIZED, FLAMMABLE, N.O.S., not refrigerated liquid |
| 2.1 |  | 3312 | GAS, REFRIGERATED LIQUID, FLAMMABLE, N.O.S. |
| 2.1 |  | 3501 | CHEMICAL UNDER PRESSURE, FLAMMABLE, N.O.S. |
| 2.1 |  | 3510 | ADSORBED GAS, FLAMMABLE, N.O.S. |
| 2.1 | See 2.0.6.6 | 3537 | ARTICLES CONTAINING FLAMMABLE GAS, N.O.S. |
| 2.1 | 6.1 | 3504 | CHEMICAL UNDER PRESSURE, FLAMMABLE, TOXIC, N.O.S. |
| 2.1 | 8 | 3505 | CHEMICAL UNDER PRESSURE, FLAMMABLE, CORROSIVE, N.O.S. |
|  |  |  | Class 2.2 |
|  |  |  | Specific entries |
| 2.2 |  | 1078 | REFRIGERANT GAS, N.O.S. |
| 2.2 |  | 1968 | INSECTICIDE GAS, N.O.S. |
|  |  |  | General entries |
| 2.2 |  | 1956 | COMPRESSED GAS, N.O.S. |
| 2.2 |  | 3163 | LIQUEFIED GAS, N.O.S. |
| 2.2 |  | 3158 | GAS, REFRIGERATED LIQUID, N.O.S. |
| 2.2 |  | 3500 | CHEMICAL UNDER PRESSURE, N.O.S. |
| 2.2 |  | 3511 | ADSORBED GAS, N.O.S. |
| 2.2 | See 2.0.6.6 | 3538 | ARTICLES CONTAINING NON-FLAMMABLE, NON-TOXIC GAS, N.O.S. |
| 2.2 | 5.1 | 3156 | COMPRESSED GAS, OXIDIZING, N.O.S. |
| 2.2 | 5.1 | 3157 | LIQUEFIED GAS, OXIDIZING, N.O.S. |
| 2.2 | 5.1 | 3311 | GAS, REFRIGERATED LIQUID, OXIDIZING, N.O.S. |
| 2.2 | 5.1 | 3513 | ADSORBED GAS, OXIDIZING, N.O.S. |
| 2.2 | 6.1 | 3502 | CHEMICAL UNDER PRESSURE, TOXIC, N.O.S. |
| 2.2 | 8 | 3503 | CHEMICAL UNDER PRESSURE, CORROSIVE, N.O.S. |
|  |  |  | Class 2.3 |
|  |  |  | Specific entries |
| 2.3 |  | 1967 | INSECTICIDE GAS, TOXIC, N.O.S. |
| 2.3 | 2.1 | 3355 | INSECTICIDE GAS, TOXIC, FLAMMABLE, N.O.S. |
|  |  |  | General entries |
| 2.3 |  | 1955 | COMPRESSED GAS, TOXIC, N.O.S. |
| 2.3 |  | 3162 | LIQUEFIED GAS, TOXIC, N.O.S. |
| 2.3 |  | 3169 | GAS SAMPLE, NON-PRESSURIZED, TOXIC, N.O.S., not refrigerated liquid |
| 2.3 |  | 3512 | ADSORBED GAS, TOXIC, N.O.S. |
| 2.3 | See 2.0.6.6 | 3539 | ARTICLES CONTAINING TOXIC GAS, N.O.S. |
| 2.3 | 2.1 | 1953 | COMPRESSED GAS, TOXIC, FLAMMABLE, N.O.S. |
| 2.3 | 2.1 | 3160 | LIQUEFIED GAS, TOXIC, FLAMMABLE, N.O.S. |
| 2.3 | 2.1 | 3168 | GAS SAMPLE, NON-PRESSURIZED, TOXIC, FLAMMABLE, N.O.S., not refrigerated liquid |
| 2.3 | 2.1 | 3514 | ADSORBED GAS, TOXIC, FLAMMABLE, N.O.S. |
| 2.3 | 2.1 + 8 | 3305 | COMPRESSED GAS, TOXIC, FLAMMABLE, CORROSIVE, N.O.S. |
| 2.3 | 2.1 + 8 | 3309 | LIQUEFIED GAS, TOXIC, FLAMMABLE, CORROSIVE, N.O.S. |
| 2.3 | 2.1 + 8 | 3517 | ADSORBED GAS, TOXIC, FLAMMABLE, CORROSIVE, N.O.S. |
| 2.3 | 5.1 | 3303 | COMPRESSED GAS, TOXIC, OXIDIZING, N.O.S. |
| 2.3 | 5.1 | 3307 | LIQUEFIED GAS, TOXIC, OXIDIZING, N.O.S. |
| 2.3 | 5.1 | 3515 | ADSORBED GAS, TOXIC, OXIDIZING, N.O.S. |
| 2.3 | 5.1 + 8 | 3306 | COMPRESSED GAS, TOXIC, OXIDIZING, CORROSIVE, N.O.S. |

APPENDIX A

| Class or division | Subsidiary hazard | UN No. | Proper shipping name |
| --- | --- | --- | --- |
|  |  |  | CLASS 2 (continued) |
|  |  |  | Class 2.3 (continued) |
| 2.3 | 5.1 + 8 | 3310 | LIQUEFIED GAS, TOXIC, OXIDIZING, CORROSIVE, N.O.S. |
| 2.3 | 5.1 + 8 | 3518 | ADSORBED GAS, TOXIC, OXIDIZING, CORROSIVE, N.O.S. |
| 2.3 | 8 | 3304 | COMPRESSED GAS, TOXIC, CORROSIVE, N.O.S. |
| 2.3 | 8 | 3308 | LIQUEFIED GAS, TOXIC, CORROSIVE, N.O.S. |
| 2.3 | 8 | 3516 | ADSORBED GAS, TOXIC, CORROSIVE, N.O.S. |

APPENDIX A

| Class or division | Subsidiary hazard | UN No. | Proper shipping name |
| --- | --- | --- | --- |
|  |  |  | CLASS 3 |
|  |  |  | Specific entries |
| 3 |  | 1224 | KETONES, LIQUID, N.O.S. |
| 3 |  | 1268 | PETROLEUM DISTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. |
| 3 |  | 1987 | ALCOHOLS, N.O.S. |
| 3 |  | 1989 | ALDEHYDES, N.O.S. |
| 3 |  | 2319 | TERPENE HYDROCARBONS, N.O.S. |
| 3 |  | 3271 | ETHERS, N.O.S. |
| 3 |  | 3272 | ESTERS, N.O.S. |
| 3 |  | 3295 | HYDROCARBONS, LIQUID, N.O.S. |
| 3 |  | 3336 | MERCAPTANS, LIQUID, FLAMMABLE, N.O.S. or MERCAPTAN MIXTURE, LIQUID, FLAMMABLE, N.O.S. |
| 3 |  | 3343 | NITROGLYCERIN MIXTURE, DESENSITIZED, LIQUID, FLAMMABLE, N.O.S. with not more than 30% nitroglycerin, by mass |
| 3 |  | 3357 | NITROGLYCERIN MIXTURE, DESENSITIZED, LIQUID, N.O.S. with not more than 30% nitroglycerin, by mass |
| 3 |  | 3379 | DESENSITIZED EXPLOSIVE, LIQUID, N.O.S. |
| 3 | 6.1 | 1228 | MERCAPTANS, LIQUID, FLAMMABLE, TOXIC, N.O.S. or MERCAPTAN MIXTURE, LIQUID, FLAMMABLE, TOXIC, N.O.S. |
| 3 | 6.1 | 1986 | ALCOHOLS, FLAMMABLE, TOXIC, N.O.S. |
| 3 | 6.1 | 1988 | ALDEHYDES, FLAMMABLE, TOXIC, N.O.S. |
| 3 | 6.1 | 2478 | ISOCYANATES, FLAMMABLE, TOXIC, N.O.S. or ISOCYANATE SOLUTION, FLAMMABLE, TOXIC, N.O.S. |
| 3 | 6.1 | 3248 | MEDICINE, LIQUID, FLAMMABLE, TOXIC, N.O.S. |
| 3 | 6.1 | 3273 | NITRILES, FLAMMABLE, TOXIC, N.O.S. |
| 3 | 8 | 2733 | AMINES, FLAMMABLE, CORROSIVE, N.O.S. or POLYAMINES, FLAMMABLE, CORROSIVE, N.O.S. |
| 3 | 8 | 2985 | CHLOROSILANES, FLAMMABLE, CORROSIVE, N.O.S. |
| 3 | 8 | 3274 | ALCOHOLATES SOLUTION, N.O.S. in alcohol |
|  |  |  | Pesticides |
| 3 | 6.1 | 2758 | CARBAMATE PESTICIDE, LIQUID, FLAMMABLE, TOXIC flashpoint < 23°C |
| 3 | 6.1 | 2760 | ARSENICAL PESTICIDE, LIQUID, FLAMMABLE, TOXIC flashpoint < 23°C |
| 3 | 6.1 | 2762 | ORGANOCHLORINE PESTICIDE, LIQUID, FLAMMABLE, TOXIC flashpoint < 23°C |
| 3 | 6.1 | 2764 | TRIAZINE PESTICIDE, LIQUID, FLAMMABLE, TOXIC flashpoint < 23°C |
| 3 | 6.1 | 2772 | THIOCARBAMATE PESTICIDE, LIQUID, FLAMMABLE, TOXIC flashpoint < 23°C |
| 3 | 6.1 | 2776 | COPPER BASED PESTICIDE, LIQUID, FLAMMABLE, TOXIC flashpoint < 23°C |
| 3 | 6.1 | 2778 | MERCURY BASED PESTICIDE, LIQUID, FLAMMABLE, TOXIC flashpoint < 23°C |
| 3 | 6.1 | 2780 | SUBSTITUTED NITROPHENOL PESTICIDE, LIQUID, FLAMMABLE, TOXIC flashpoint < 23°C |
| 3 | 6.1 | 2782 | BIPYRIDILIUM PESTICIDE, LIQUID, FLAMMABLE, TOXIC flashpoint < 23°C |
| 3 | 6.1 | 2784 | ORGANOPHOSPHORUS PESTICIDE, LIQUID, FLAMMABLE, TOXIC flashpoint < 23°C |
| 3 | 6.1 | 2787 | ORGANOTIN PESTICIDE, LIQUID, FLAMMABLE, TOXIC flashpoint < 23°C |
| 3 | 6.1 | 3021 | PESTICIDE, LIQUID, FLAMMABLE, TOXIC, N.O.S. flashpoint < 23°C |
| 3 | 6.1 | 3024 | COUMARIN DERIVATIVE PESTICIDE, LIQUID, FLAMMABLE, TOXIC flashpoint < 23°C |
| 3 | 6.1 | 3346 | PHENOXYACETIC ACID DERIVATIVE PESTICIDE, LIQUID, FLAMMABLE, TOXIC flashpoint < 23°C |
| 3 | 6.1 | 3350 | PYRETHROID PESTICIDE, LIQUID, FLAMMABLE, TOXIC flashpoint < 23°C |
|  |  |  | General entries |
| 3 |  | 1993 | FLAMMABLE LIQUID, N.O.S. |
| 3 |  | 3256 | ELEVATED TEMPERATURE LIQUID, FLAMMABLE, N.O.S. with flashpoint above 60°C, at or above its flashpoint |
| 3 | See 2.0.6.6 | 3540 | ARTICLES CONTAINING FLAMMABLE LIQUID, N.O.S. |
| 3 | 6.1 | 1992 | FLAMMABLE LIQUID, TOXIC, N.O.S. |
| 3 | 6.1 + 8 | 3286 | FLAMMABLE LIQUID, TOXIC, CORROSIVE, N.O.S. |
| 3 | 8 | 2924 | FLAMMABLE LIQUID, CORROSIVE, N.O.S. |

APPENDIX A

| Class or division | Subsidiary hazard | UN No. | Proper shipping name |
| --- | --- | --- | --- |
|  |  |  | CLASS 4 |
|  |  |  | Class 4.1 |
|  |  |  | Specific entries |
| 4.1 |  | 1353 | FIBRES or FABRICS IMPREGNATED WITH WEAKLY NITRATED NITROCELLULOSE, N.O.S. |
| 4.1 |  | 3089 | METAL POWDER, FLAMMABLE, N.O.S. |
| 4.1 |  | 3182 | METAL HYDRIDES, FLAMMABLE, N.O.S. |
| 4.1 |  | 3221 | SELF-REACTIVE LIQUID TYPE B |
| 4.1 |  | 3222 | SELF-REACTIVE SOLID TYPE B |
| 4.1 |  | 3223 | SELF-REACTIVE LIQUID TYPE C |
| 4.1 |  | 3224 | SELF-REACTIVE SOLID TYPE C |
| 4.1 |  | 3225 | SELF-REACTIVE LIQUID TYPE D |
| 4.1 |  | 3226 | SELF-REACTIVE SOLID TYPE D |
| 4.1 |  | 3227 | SELF-REACTIVE LIQUID TYPE E |
| 4.1 |  | 3228 | SELF-REACTIVE SOLID TYPE E |
| 4.1 |  | 3229 | SELF-REACTIVE LIQUID TYPE F |
| 4.1 |  | 3230 | SELF-REACTIVE SOLID TYPE F |
| 4.1 |  | 3231 | SELF-REACTIVE LIQUID TYPE B, TEMPERATURE CONTROLLED |
| 4.1 |  | 3232 | SELF-REACTIVE SOLID TYPE B, TEMPERATURE CONTROLLED |
| 4.1 |  | 3233 | SELF-REACTIVE LIQUID TYPE C, TEMPERATURE CONTROLLED |
| 4.1 |  | 3234 | SELF-REACTIVE SOLID TYPE C, TEMPERATURE CONTROLLED |
| 4.1 |  | 3235 | SELF-REACTIVE LIQUID TYPE D, TEMPERATURE CONTROLLED |
| 4.1 |  | 3236 | SELF-REACTIVE SOLID TYPE D, TEMPERATURE CONTROLLED |
| 4.1 |  | 3237 | SELF-REACTIVE LIQUID TYPE E, TEMPERATURE CONTROLLED |
| 4.1 |  | 3238 | SELF-REACTIVE SOLID TYPE E, TEMPERATURE CONTROLLED |
| 4.1 |  | 3239 | SELF-REACTIVE LIQUID TYPE F, TEMPERATURE CONTROLLED |
| 4.1 |  | 3240 | SELF-REACTIVE SOLID TYPE F, TEMPERATURE CONTROLLED |
| 4.1 |  | 3319 | NITROGLYCERIN MIXTURE, DESENSITIZED, SOLID, N.O.S. with more than 2% but not more than 10% nitroglycerin, by mass |
| 4.1 |  | 3344 | PENTAERYTHRITE TETRANITRATE (PENTAERYTHRITOL TETRANITRATE; PETN) MIXTURE, DESENSITIZED, SOLID, N.O.S. with more than 10% but not more than 20% PETN, by mass |
| 4.1 |  | 3380 | DESENSITIZED EXPLOSIVE, SOLID, N.O.S. |
| 4.1 |  | 3531 | POLYMERIZING SUBSTANCE, SOLID, STABILIZED, N.O.S. |
| 4.1 |  | 3532 | POLYMERIZING SUBSTANCE, LIQUID, STABILIZED, N.O.S. |
| 4.1 |  | 3533 | POLYMERIZING SUBSTANCE, SOLID, TEMPERATURE CONTROLLED, N.O.S. |
| 4.1 |  | 3534 | POLYMERIZING SUBSTANCE, LIQUID, TEMPERATURE CONTROLLED, N.O.S. |
|  |  |  | General entries |
| 4.1 |  | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 4.1 |  | 3175 | SOLIDS CONTAINING FLAMMABLE LIQUID, N.O.S. |
| 4.1 |  | 3176 | FLAMMABLE SOLID, ORGANIC, MOLTEN, N.O.S. |
| 4.1 |  | 3178 | FLAMMABLE SOLID, INORGANIC, N.O.S. |
| 4.1 |  | 3181 | METAL SALTS OF ORGANIC COMPOUNDS, FLAMMABLE, N.O.S. |
| 4.1 | See 2.0.6.6 | 3541 | ARTICLES CONTAINING FLAMMABLE SOLID, N.O.S. |
| 4.1 | 5.1 | 3097 | FLAMMABLE SOLID, OXIDIZING, N.O.S. |
| 4.1 | 6.1 | 2926 | FLAMMABLE SOLID, TOXIC, ORGANIC, N.O.S. |
| 4.1 | 6.1 | 3179 | FLAMMABLE SOLID, TOXIC, INORGANIC, N.O.S. |
| 4.1 | 8 | 2925 | FLAMMABLE SOLID, CORROSIVE, ORGANIC, N.O.S. |
| 4.1 | 8 | 3180 | FLAMMABLE SOLID, CORROSIVE, INORGANIC, N.O.S. |
| 4.2 4.2 4.2 4.2 4.2 4.2 |  | 1373 1378 1383 2006 2881 3189 | Class 4.2 Specific entries FIBRES or FABRICS, ANIMAL or VEGETABLE or SYNTHETIC, N.O.S. with oil METAL CATALYST, WETTED with a visible excess of liquid PYROPHORIC METAL, N.O.S. or PYROPHORIC ALLOY, N.O.S. PLASTICS, NITROCELLULOSE-BASED, SELF-HEATING, N.O.S. METAL CATALYST, DRY METAL POWDER, SELF-HEATING, N.O.S. |

APPENDIX A

| Class or division | Subsidiary hazard | UN No. | Proper shipping name |
| --- | --- | --- | --- |
|  |  |  | CLASS 4 (continued) |
|  |  |  | Class 4.2 (continued) |
|  |  |  | Specific entries (continued) |
| 4.2 |  | 3205 | ALKALINE EARTH METAL ALCOHOLATES, N.O.S. |
| 4.2 |  | 3313 | ORGANIC PIGMENTS, SELF-HEATING |
| 4.2 |  | 3342 | XANTHATES |
| 4.2 |  | 3391 | ORGANOMETALLIC SUBSTANCE, SOLID, PYROPHORIC |
| 4.2 |  | 3392 | ORGANOMETALLIC SUBSTANCE, LIQUID, PYROPHORIC |
| 4.2 |  | 3400 | ORGANOMETALLIC SUBSTANCE, SOLID, SELF-HEATING |
| 4.2 | 4.3 | 3393 | ORGANOMETALLIC SUBSTANCE, SOLID, PYROPHORIC, WATER-REACTIVE |
| 4.2 | 4.3 | 3394 | ORGANOMETALLIC SUBSTANCE, LIQUID, PYROPHORIC, WATER-REACTIVE |
| 4.2 | 8 | 3206 | ALKALI METAL ALCOHOLATES, SELF-HEATING, CORROSIVE, N.O.S. |
|  |  |  | General entries |
| 4.2 |  | 2845 | PYROPHORIC LIQUID, ORGANIC, N.O.S. |
| 4.2 |  | 2846 | PYROPHORIC SOLID, ORGANIC, N.O.S. |
| 4.2 |  | 3088 | SELF-HEATING SOLID, ORGANIC, N.O.S. |
| 4.2 |  | 3183 | SELF-HEATING LIQUID, ORGANIC, N.O.S. |
| 4.2 |  | 3186 | SELF-HEATING LIQUID, INORGANIC, N.O.S. |
| 4.2 |  | 3190 | SELF-HEATING SOLID, INORGANIC, N.O.S. |
| 4.2 |  | 3194 | PYROPHORIC LIQUID, INORGANIC, N.O.S. |
| 4.2 |  | 3200 | PYROPHORIC SOLID, INORGANIC, N.O.S. |
| 4.2 | See 2.0.6.6 | 3542 | ARTICLES CONTAINING A SUBSTANCE LIABLE TO SPONTANEOUS COMBUSTION, N.O.S. |
| 4.2 | 5.1 | 3127 | SELF-HEATING SOLID, OXIDIZING, N.O.S. |
| 4.2 | 6.1 | 3128 | SELF-HEATING SOLID, TOXIC, ORGANIC, N.O.S. |
| 4.2 | 6.1 | 3184 | SELF-HEATING LIQUID, TOXIC, ORGANIC, N.O.S. |
| 4.2 | 6.1 | 3187 | SELF-HEATING LIQUID, TOXIC, INORGANIC, N.O.S. |
| 4.2 | 6.1 | 3191 | SELF-HEATING SOLID, TOXIC, INORGANIC, N.O.S. |
| 4.2 | 8 | 3126 | SELF-HEATING SOLID, CORROSIVE, ORGANIC, N.O.S. |
| 4.2 | 8 | 3185 | SELF-HEATING LIQUID, CORROSIVE, ORGANIC, N.O.S. |
| 4.2 | 8 | 3188 | SELF-HEATING LIQUID, CORROSIVE, INORGANIC, N.O.S. |
| 4.2 | 8 | 3192 | SELF-HEATING SOLID, CORROSIVE, INORGANIC, N.O.S. |
|  |  |  | Class 4.3 |
|  |  |  | Specific entries |
| 4.3 |  | 1389 | ALKALI METAL AMALGAM, LIQUID |
| 4.3 |  | 1390 | ALKALI METAL AMIDES |
| 4.3 |  | 1391 | ALKALI METAL DISPERSION or ALKALINE EARTH METAL DISPERSION |
| 4.3 |  | 1392 | ALKALINE EARTH METAL AMALGAM, LIQUID |
| 4.3 |  | 1393 | ALKALINE EARTH METAL ALLOY, N.O.S. |
| 4.3 |  | 1409 | METAL HYDRIDES, WATER-REACTIVE, N.O.S. |
| 4.3 |  | 1421 | ALKALI METAL ALLOY, LIQUID, N.O.S. |
| 4.3 |  | 3208 | METALLIC SUBSTANCE, WATER-REACTIVE, N.O.S. |
| 4.3 |  | 3395 | ORGANOMETALLIC SUBSTANCE, SOLID, WATER-REACTIVE |
| 4.3 |  | 3398 | ORGANOMETALLIC SUBSTANCE, LIQUID, WATER-REACTIVE |
| 4.3 |  | 3401 | ALKALI METAL AMALGAM, SOLID |
| 4.3 |  | 3402 | ALKALINE EARTH METAL AMALGAM, SOLID |
| 4.3 | 3 | 3399 | ORGANOMETALLIC SUBSTANCE, LIQUID, WATER-REACTIVE, FLAMMABLE |
| 4.3 | 3 | 3482 | ALKALI METAL DISPERSION, FLAMMABLE or ALKALINE EARTH METAL DISPERSION, FLAMMABLE |
| 4.3 | 3 + 8 | 2988 | CHLOROSILANES, WATER-REACTIVE, FLAMMABLE, CORROSIVE, N.O.S. |
| 4.3 | 4.1 | 3396 | ORGANOMETALLIC SUBSTANCE, SOLID, WATER-REACTIVE, FLAMMABLE |
| 4.3 | 4.2 | 3209 | METALLIC SUBSTANCE, WATER-REACTIVE, SELF-HEATING, N.O.S. |
| 4.3 | 4.2 | 3397 | ORGANOMETALLIC SUBSTANCE, SOLID, WATER-REACTIVE, SELF-HEATING |
|  |  |  | General entries |
| 4.3 |  | 3148 | WATER-REACTIVE LIQUID, N.O.S. |
| 4.3 |  | 2813 | WATER-REACTIVE SOLID, N.O.S. |

APPENDIX A

| Class or division | Subsidiary hazard | UN No. | Proper shipping name |
| --- | --- | --- | --- |
|  |  |  | CLASS 4 (continued) |
|  |  |  | Class 4.3 (continued) |
| 4.3 | See 2.0.6.6 | 3543 | ARTICLES CONTAINING A SUBSTANCE WHICH IN CONTACT WITH WATER EMITS FLAMMABLE GASES, N.O.S. |
| 4.3 | 4.1 | 3132 | WATER-REACTIVE SOLID, FLAMMABLE, N.O.S. |
| 4.3 | 4.2 | 3135 | WATER-REACTIVE SOLID, SELF-HEATING, N.O.S. |
| 4.3 | 5.1 | 3133 | WATER-REACTIVE SOLID, OXIDIZING, N.O.S. |
| 4.3 | 6.1 | 3130 | WATER-REACTIVE LIQUID, TOXIC, N.O.S. |
| 4.3 | 6.1 | 3134 | WATER-REACTIVE SOLID, TOXIC, N.O.S. |
| 4.3 | 8 | 3129 | WATER-REACTIVE LIQUID, CORROSIVE, N.O.S. |
| 4.3 | 8 | 3131 | WATER-REACTIVE SOLID, CORROSIVE, N.O.S. |

APPENDIX A

| Class or division | Subsidiary hazard | UN No. | Proper shipping name |
| --- | --- | --- | --- |
|  |  |  | CLASS 5 |
|  |  |  | Class 5.1 |
|  |  |  | Specific entries |
| 5.1 |  | 1450 | BROMATES, INORGANIC, N.O.S. |
| 5.1 |  | 1461 | CHLORATES, INORGANIC, N.O.S. |
| 5.1 |  | 1462 | CHLORITES, INORGANIC, N.O.S. |
| 5.1 |  | 1477 | NITRATES, INORGANIC, N.O.S. |
| 5.1 |  | 1481 | PERCHLORATES, INORGANIC, N.O.S. |
| 5.1 |  | 1482 | PERMANGANATES, INORGANIC, N.O.S. |
| 5.1 |  | 1483 | PEROXIDES, INORGANIC, N.O.S. |
| 5.1 |  | 2627 | NITRITES, INORGANIC, N.O.S. |
| 5.1 |  | 3210 | CHLORATES, INORGANIC, AQUEOUS SOLUTION, N.O.S. |
| 5.1 |  | 3211 | PERCHLORATES, INORGANIC, AQUEOUS SOLUTION, N.O.S. |
| 5.1 |  | 3212 | HYPOCHLORITES, INORGANIC, N.O.S. |
| 5.1 |  | 3213 | BROMATES, INORGANIC, AQUEOUS SOLUTION, N.O.S. |
| 5.1 |  | 3214 | PERMANGANATES, INORGANIC, AQUEOUS SOLUTION, N.O.S. |
| 5.1 |  | 3215 | PERSULPHATES, INORGANIC, N.O.S. |
| 5.1 |  | 3216 | PERSULPHATES, INORGANIC, AQUEOUS SOLUTION, N.O.S. |
| 5.1 |  | 3218 | NITRATES, INORGANIC, AQUEOUS SOLUTION, N.O.S. |
| 5.1 |  | 3219 | NITRITES, INORGANIC, AQUEOUS SOLUTION, N.O.S. |
|  |  |  | General entries |
| 5.1 |  | 1479 | OXIDIZING SOLID, N.O.S. |
| 5.1 |  | 3139 | OXIDIZING LIQUID, N.O.S. |
| 5.1 | See 2.0.6.6 | 3544 | ARTICLES CONTAINING OXIDIZING SUBSTANCE, N.O.S. |
| 5.1 | 4.1 | 3137 | OXIDIZING SOLID, FLAMMABLE, N.O.S. |
| 5.1 | 4.2 | 3100 | OXIDIZING SOLID, SELF-HEATING, N.O.S. |
| 5.1 | 4.3 | 3121 | OXIDIZING SOLID, WATER-REACTIVE, N.O.S. |
| 5.1 | 6.1 | 3087 | OXIDIZING SOLID, TOXIC, N.O.S. |
| 5.1 | 6.1 | 3099 | OXIDIZING LIQUID, TOXIC, N.O.S. |
| 5.1 | 8 | 3085 | OXIDIZING SOLID, CORROSIVE, N.O.S. |
| 5.1 | 8 | 3098 | OXIDIZING LIQUID, CORROSIVE, N.O.S. |
|  |  |  | Class 5.2 |
|  |  |  | Specific entries |
| 5.2 |  | 3101 | ORGANIC PEROXIDE TYPE B, LIQUID |
| 5.2 |  | 3102 | ORGANIC PEROXIDE TYPE B, SOLID |
| 5.2 |  | 3103 | ORGANIC PEROXIDE TYPE C, LIQUID |
| 5.2 |  | 3104 | ORGANIC PEROXIDE TYPE C, SOLID |
| 5.2 |  | 3105 | ORGANIC PEROXIDE TYPE D, LIQUID |
| 5.2 |  | 3106 | ORGANIC PEROXIDE TYPE D, SOLID |
| 5.2 |  | 3107 | ORGANIC PEROXIDE TYPE E, LIQUID |
| 5.2 |  | 3108 | ORGANIC PEROXIDE TYPE E, SOLID |
| 5.2 |  | 3109 | ORGANIC PEROXIDE TYPE F, LIQUID |
| 5.2 |  | 3110 | ORGANIC PEROXIDE TYPE F, SOLID |
| 5.2 |  | 3111 | ORGANIC PEROXIDE TYPE B, LIQUID, TEMPERATURE CONTROLLED |
| 5.2 |  | 3112 | ORGANIC PEROXIDE TYPE B, SOLID, TEMPERATURE CONTROLLED |
| 5.2 |  | 3113 | ORGANIC PEROXIDE TYPE C, LIQUID, TEMPERATURE CONTROLLED |
| 5.2 |  | 3114 | ORGANIC PEROXIDE TYPE C, SOLID, TEMPERATURE CONTROLLED |
| 5.2 |  | 3115 | ORGANIC PEROXIDE TYPE D, LIQUID, TEMPERATURE CONTROLLED |
| 5.2 |  | 3116 | ORGANIC PEROXIDE TYPE D, SOLID, TEMPERATURE CONTROLLED |
| 5.2 |  | 3117 | ORGANIC PEROXIDE TYPE E, LIQUID, TEMPERATURE CONTROLLED |
| 5.2 |  | 3118 | ORGANIC PEROXIDE TYPE E, SOLID, TEMPERATURE CONTROLLED |
| 5.2 |  | 3119 | ORGANIC PEROXIDE TYPE F, LIQUID, TEMPERATURE CONTROLLED |
| 5.2 |  | 3120 | ORGANIC PEROXIDE TYPE F, SOLID, TEMPERATURE CONTROLLED |
|  |  |  | General entries |
| 5.2 | See 2.0.6.6 | 3545 | ARTICLES CONTAINING ORGANIC PEROXIDE, N.O.S. |

APPENDIX A

| Class or division | Subsidiary hazard | UN No. | Proper shipping name |
| --- | --- | --- | --- |
|  |  |  | CLASS 6 |
|  |  |  | Class 6.1 |
|  |  |  | Specific entries |
| 6.1 |  | 1544 | ALKALOIDS, SOLID, N.O.S. or ALKALOID SALTS, SOLID, N.O.S. |
| 6.1 |  | 1549 | ANTIMONY COMPOUND, INORGANIC, SOLID, N.O.S. |
| 6.1 |  | 1556 | ARSENIC COMPOUND, LIQUID, N.O.S. inorganic, including: Arsenates, n.o.s., Arsenites, n.o.s., and Arsenic sulphides, n.o.s. |
| 6.1 |  | 1557 | ARSENIC COMPOUND, SOLID, N.O.S. inorganic, including: Arsenates, n.o.s., Arsenites, n.o.s., and Arsenic sulphides, n.o.s. |
| 6.1 |  | 1564 | BARIUM COMPOUND, N.O.S. |
| 6.1 |  | 1566 | BERYLLIUM COMPOUND, N.O.S. |
| 6.1 |  | 1583 | CHLOROPICRIN MIXTURE, N.O.S. |
| 6.1 |  | 1588 | CYANIDES, INORGANIC, SOLID, N.O.S. |
| 6.1 |  | 1601 | DISINFECTANT, SOLID, TOXIC, N.O.S. |
| 6.1 |  | 1602 | DYE, LIQUID, TOXIC, N.O.S. or DYE INTERMEDIATE, LIQUID, TOXIC, N.O.S. |
| 6.1 |  | 1655 | NICOTINE COMPOUND, SOLID, N.O.S. or NICOTINE PREPARATION, SOLID, N.O.S. |
| 6.1 |  | 1693 | TEAR GAS SUBSTANCE, LIQUID, N.O.S. |
| 6.1 |  | 1707 | THALLIUM COMPOUND, N.O.S. |
| 6.1 |  | 1851 | MEDICINE, LIQUID, TOXIC, N.O.S. |
| 6.1 |  | 1935 | CYANIDE SOLUTION, N.O.S. |
| 6.1 |  | 2024 | MERCURY COMPOUND, LIQUID, N.O.S. |
| 6.1 |  | 2025 | MERCURY COMPOUND, SOLID, N.O.S. |
| 6.1 |  | 2026 | PHENYLMERCURIC COMPOUND, N.O.S. |
| 6.1 |  | 2206 | ISOCYANATES, TOXIC, N.O.S. or ISOCYANATE SOLUTION, TOXIC, N.O.S. |
| 6.1 |  | 2291 | LEAD COMPOUND, SOLUBLE, N.O.S. |
| 6.1 |  | 2570 | CADMIUM COMPOUND |
| 6.1 |  | 2788 | ORGANOTIN COMPOUND, LIQUID, N.O.S. |
| 6.1 |  | 2856 | FLUOROSILICATES, N.O.S. |
| 6.1 |  | 3140 | ALKALOIDS, LIQUID, N.O.S or ALKALOIDS SALTS, LIQUID, N.O.S. |
| 6.1 |  | 3141 | ANTIMONY COMPOUND, INORGANIC, LIQUID, N.O.S. |
| 6.1 |  | 3142 | DISINFECTANT, LIQUID, TOXIC, N.O.S. |
| 6.1 |  | 3143 | DYE, SOLID, TOXIC, N.O.S. or DYE INTERMEDIATE, SOLID, TOXIC, N.O.S. |
| 6.1 |  | 3144 | NICOTINE COMPOUND, LIQUID, N.O.S. or NICOTINE PREPARATION, LIQUID, N.O.S. |
| 6.1 |  | 3146 | ORGANOTIN COMPOUND, SOLID, N.O.S. |
| 6.1 |  | 3249 | MEDICINE, SOLID, TOXIC, N.O.S. |
| 6.1 |  | 3276 | NITRILES, TOXIC, LIQUID, N.O.S. |
| 6.1 |  | 3278 | ORGANOPHOSPHORUS COMPOUND, TOXIC, LIQUID, N.O.S. |
| 6.1 |  | 3280 | ORGANOARSENIC COMPOUND, LIQUID, N.O.S. |
| 6.1 |  | 3281 | METAL CARBONYLS, LIQUID, N.O.S. with LC50 |
| 6.1 |  | 3282 | ORGANOMETALLIC COMPOUND, TOXIC, LIQUID, N.O.S. with LC50 |
| 6.1 |  | 3283 | SELENIUM COMPOUND, SOLID, N.O.S. with LC50 |
| 6.1 |  | 3284 | TELLURIUM COMPOUND, N.O.S. with LC50 |
| 6.1 |  | 3285 | VANADIUM COMPOUND, N.O.S. |
| 6.1 |  | 3439 | NITRILES, TOXIC, SOLID, N.O.S. |
| 6.1 |  | 3440 | SELENIUM COMPOUND, LIQUID, N.O.S. |
| 6.1 |  | 3448 | TEAR GAS SUBSTANCE, SOLID, N.O.S. |
| 6.1 |  | 3464 | ORGANOPHOSPHORUS COMPOUND, TOXIC, SOLID, N.O.S. |
| 6.1 |  | 3465 | ORGANOARSENIC COMPOUND, SOLID, N.O.S. |
| 6.1 |  | 3466 | METAL CARBONYLS, SOLID, N.O.S. |
| 6.1 |  | 3467 | ORGANOMETALLIC COMPOUND, TOXIC, SOLID, N.O.S. |
| 6.1 | 3 | 3071 | MERCAPTANS, LIQUID, TOXIC, FLAMMABLE, N.O.S. or MERCAPTAN MIXTURE, LIQUID, TOXIC, FLAMMABLE, N.O.S. |
| 6.1 | 3 | 3080 | ISOCYANATES, TOXIC, FLAMMABLE, N.O.S or ISOCYANATE SOLUTION, TOXIC, FLAMMABLE, N.O.S. |
| 6.1 | 3 | 3275 | NITRILES, TOXIC, FLAMMABLE, N.O.S. |
| 6.1 | 3 | 3279 | ORGANOPHOSPHORUS COMPOUND, TOXIC, FLAMMABLE, N.O.S. |
| 6.1 | 3 + 8 | 2742 | CHLOROFORMATES, TOXIC, CORROSIVE, FLAMMABLE, N.O.S. |

APPENDIX A

| Class or division | Subsidiary hazard | UN No. | Proper shipping name |
| --- | --- | --- | --- |
|  |  |  | CLASS 6 (continued) |
|  |  |  | Class 6.1 (continued) |
|  |  |  | Specific entries (continued) |
| 6.1 | 3 + 8 | 3362 | CHLOROSILANES, TOXIC, CORROSIVE, FLAMMABLE, N.O.S. |
| 6.1 | 8 | 3277 | CHLOROFORMATES, TOXIC, CORROSIVE, N.O.S. |
| 6.1 | 8 | 3361 | CHLOROSILANES, TOXIC, CORROSIVE, N.O.S. |
|  |  |  | Pesticides |
|  |  |  | (a) Solid |
| 6.1 |  | 2588 | PESTICIDE, SOLID, TOXIC, N.O.S. |
| 6.1 |  | 2757 | CARBAMATE PESTICIDE, SOLID, TOXIC |
| 6.1 |  | 2759 | ARSENICAL PESTICIDE, SOLID, TOXIC |
| 6.1 |  | 2761 | ORGANOCHLORINE PESTICIDE, SOLID, TOXIC |
| 6.1 |  | 2763 | TRIAZINE PESTICIDE, SOLID, TOXIC |
| 6.1 |  | 2771 | THIOCARBAMATE PESTICIDE, SOLID, TOXIC |
| 6.1 |  | 2775 | COPPER BASED PESTICIDE, SOLID, TOXIC |
| 6.1 |  | 2777 | MERCURY BASED PESTICIDE, SOLID, TOXIC |
| 6.1 |  | 2779 | SUBSTITUTED NITROPHENOL PESTICIDE, SOLID, TOXIC |
| 6.1 |  | 2781 | BIPYRIDILIUM PESTICIDE, SOLID, TOXIC |
| 6.1 |  | 2783 | ORGANOPHOSPHORUS PESTICIDE, SOLID, TOXIC |
| 6.1 |  | 2786 | ORGANOTIN PESTICIDE, SOLID, TOXIC |
| 6.1 |  | 3027 | COUMARIN DERIVATIVE PESTICIDE, SOLID, TOXIC |
| 6.1 |  | 3345 | PHENOXYACETIC ACID DERIVATIVE PESTICIDE, SOLID, TOXIC |
| 6.1 |  | 3349 | PYRETHROID PESTICIDE, SOLID, TOXIC |
|  |  |  | (b) Liquid |
| 6.1 |  | 2902 | PESTICIDE, LIQUID TOXIC, N.O.S. |
| 6.1 |  | 2992 | CARBAMATE PESTICIDE, LIQUID, TOXIC |
| 6.1 |  | 2994 | ARSENICAL PESTICIDE, LIQUID, TOXIC |
| 6.1 |  | 2996 | ORGANOCHLORINE PESTICIDE, LIQUID, TOXIC |
| 6.1 |  | 2998 | TRIAZINE PESTICIDE, LIQUID, TOXIC |
| 6.1 |  | 3006 | THIOCARBAMATE PESTICIDE, LIQUID, TOXIC |
| 6.1 |  | 3010 | COPPER BASED PESTICIDE, LIQUID, TOXIC |
| 6.1 |  | 3012 | MERCURY BASED PESTICIDE, LIQUID, TOXIC |
| 6.1 |  | 3014 | SUBSTITUTED NITROPHENOL PESTICIDE, LIQUID, TOXIC |
| 6.1 |  | 3016 | BIPYRIDILIUM PESTICIDE, LIQUID, TOXIC |
| 6.1 |  | 3018 | ORGANOPHOSPHORUS PESTICIDE, LIQUID, TOXIC |
| 6.1 |  | 3020 | ORGANOTIN PESTICIDE, LIQUID, TOXIC |
| 6.1 |  | 3026 | COUMARIN DERIVATIVE PESTICIDE, LIQUID, TOXIC |
| 6.1 |  | 3348 | PHENOXYACETIC ACID DERIVATIVE PESTICIDE, LIQUID, TOXIC |
| 6.1 |  | 3352 | PYRETHROID PESTICIDE, LIQUID, TOXIC |
| 6.1 | 3 | 2903 | PESTICIDE, LIQUID, TOXIC, FLAMMABLE, N.O.S., flashpoint ≥ 23°C |
| 6.1 | 3 | 2991 | CARBAMATE PESTICIDE, LIQUID, TOXIC, FLAMMABLE, flashpoint ≥ 23°C |
| 6.1 | 3 | 2993 | ARSENICAL PESTICIDE, LIQUID, TOXIC, FLAMMABLE flashpoint ≥ 23°C |
| 6.1 | 3 | 2995 | ORGANOCHLORINE PESTICIDE, LIQUID, TOXIC, FLAMMABLE flashpoint ≥ 23°C |
| 6.1 | 3 | 2997 | TRIAZINE PESTICIDE, LIQUID, TOXIC, FLAMMABLE, flashpoint ≥ 23°C |
| 6.1 | 3 | 3005 | THIOCARBAMATE PESTICIDE, LIQUID, TOXIC, FLAMMABLE flashpoint ≥ 23°C |
| 6.1 | 3 | 3009 | COPPER BASED PESTICIDE, LIQUID, TOXIC, FLAMMABLE flashpoint ≥ 23°C |
| 6.1 | 3 | 3011 | MERCURY BASED PESTICIDE, LIQUID, TOXIC, FLAMMABLE flashpoint ≥ 23°C |
| 6.1 | 3 | 3013 | SUBSTITUTED NITROPHENOL PESTICIDE, LIQUID, TOXIC, FLAMMABLE flashpoint ≥ 23°C |
| 6.1 | 3 | 3015 | BIPYRIDILIUM PESTICIDE, LIQUID, TOXIC, FLAMMABLE flashpoint ≥ 23°C |
| 6.1 | 3 | 3017 | ORGANOPHOSPHORUS PESTICIDE, LIQUID, TOXIC, FLAMMABLE flashpoint ≥ 23°C |
| 6.1 | 3 | 3019 | ORGANOTIN PESTICIDE, LIQUID, TOXIC, FLAMMABLE flashpoint ≥ 23°C |
| 6.1 | 3 | 3025 | COUMARIN DERIVATIVE PESTICIDE, LIQUID, TOXIC, FLAMMABLE flashpoint ≥ 23°C |
| 6.1 | 3 | 3347 | PHENOXYACETIC ACID DERIVATIVE PESTICIDE, LIQUID, TOXIC, FLAMMABLE flashpoint ≥ 23°C |
| 6.1 | 3 | 3351 | PYRETHROID PESTICIDE, LIQUID, TOXIC, FLAMMABLE flashpoint ≥ 23°C |

APPENDIX A

| Class or division | Subsidiary hazard | UN No. | Proper shipping name |
| --- | --- | --- | --- |
|  |  |  | CLASS 6 (continued) |
|  |  |  | Class 6.1 (continued) |
|  |  |  | General entries |
| 6.1 |  | 2810 | TOXIC LIQUID, ORGANIC, N.O.S. |
| 6.1 |  | 2811 | TOXIC SOLID, ORGANIC, N.O.S. |
| 6.1 |  | 3172 | TOXINS, EXTRACTED FROM LIVING SOURCES, LIQUID, N.O.S. |
| 6.1 |  | 3243 | SOLIDS CONTAINING TOXIC LIQUID, N.O.S. |
| 6.1 |  | 3287 | TOXIC LIQUID, INORGANIC, N.O.S. |
| 6.1 |  | 3288 | TOXIC SOLID, INORGANIC, N.O.S. |
| 6.1 |  | 3315 | CHEMICAL SAMPLE, TOXIC |
| 6.1 |  | 3381 | TOXIC BY INHALATION LIQUID, N.O.S. with an LC50 lower than or equal to |
| 6.1 |  | 3382 | 200 mL/m3 and saturated vapour concentration greater than or equal to 500 LC50 TOXIC BY INHALATION LIQUID, N.O.S. with an LC50 lower than or equal to |
| 6.1 |  | 3462 | 1,000 mL/m3 and saturated vapour concentration greater than or equal to 10 LC50 TOXINS, EXTRACTED FROM LIVING SOURCES, SOLID, N.O.S. |
| 6.1 | See 2.0.6.6 | 3546 | ARTICLES CONTAINING TOXIC SUBSTANCE, N.O.S. |
| 6.1 | 3 | 2929 | TOXIC LIQUID, FLAMMABLE, ORGANIC, N.O.S. |
| 6.1 | 3 | 3383 | TOXIC BY INHALATION LIQUID, FLAMMABLE, N.O.S. with an LC50 lower than or equal to 200 mL/m3 and saturated vapour concentration greater than or equal to 500 LC50 |
| 6.1 | 3 | 3384 | TOXIC BY INHALATION LIQUID, FLAMMABLE, N.O.S. with an LC50 lower than or equal to 1,000 mL/m3 and saturated vapour concentration greater than or equal to 10 LC50 |
| 6.1 | 3 + 8 | 3488 | TOXIC BY INHALATION LIQUID, FLAMMABLE, CORROSIVE, N.O.S. with an LC50 lower than or equal to 200 mL/m3 and saturated vapour concentration greater than or equal to 500 LC50 |
| 6.1 | 3 + 8 | 3489 | TOXIC BY INHALATION LIQUID, FLAMMABLE, CORROSIVE, N.O.S. with an LC50 lower than or equal to 1,000 mL/m3 and saturated vapour concentration greater than or equal to 10 LC50 |
| 6.1 | 4.1 | 2930 | TOXIC SOLID, FLAMMABLE, ORGANIC, N.O.S. |
| 6.1 | 4.1 | 3535 | TOXIC SOLID, FLAMMABLE, INORGANIC, N.O.S. |
| 6.1 | 4.2 | 3124 | TOXIC SOLID, SELF-HEATING, N.O.S. |
| 6.1 | 4.3 | 3123 | TOXIC LIQUID, WATER-REACTIVE, N.O.S. |
| 6.1 | 4.3 | 3125 | TOXIC SOLID, WATER-REACTIVE, N.O.S. |
| 6.1 | 4.3 | 3385 | TOXIC BY INHALATION LIQUID, WATER-REACTIVE, N.O.S. with an LC50 lower than or equal to 200 mL/m3 and saturated vapour concentration greater than or equal to 500 LC50 |
| 6.1 | 4.3 | 3386 | TOXIC BY INHALATION LIQUID, WATER-REACTIVE, N.O.S. with an LC50 lower than or equal to 1,000 mL/m3 and saturated vapour concentration greater than or equal to 10 LC50 |
| 6.1 | 4.3 + 3 | 3490 | TOXIC BY INHALATION LIQUID, WATER-REACTIVE, FLAMMABLE, N.O.S. with an LC50 lower than or equal to 200 mL/m3 and saturated vapour concentration greater than or equal to 500 LC50 |
| 6.1 | 4.3 + 3 | 3491 | TOXIC BY INHALATION LIQUID, WATER-REACTIVE, FLAMMABLE, N.O.S. with an LC50 lower than or equal to 1,000 mL/m3 and saturated vapour concentration greater than or equal to 10 LC50 |
| 6.1 | 5.1 | 3122 | TOXIC LIQUID, OXIDIZING, N.O.S. |
| 6.1 | 5.1 | 3086 | TOXIC SOLID, OXIDIZING, N.O.S. |
| 6.1 | 5.1 | 3387 | TOXIC BY INHALATION LIQUID, OXIDIZING, N.O.S. with an LC50 lower than or equal to 200 mL/m3 and saturated vapour concentration greater than or equal to 500 LC50 |
| 6.1 | 5.1 | 3388 | TOXIC BY INHALATION LIQUID, OXIDIZING, N.O.S. with an LC50 lower than or equal to 1,000 mL/m3 and saturated vapour concentration greater than or equal to 10 LC50 |
| 6.1 | 8 | 2927 | TOXIC LIQUID, CORROSIVE, ORGANIC, N.O.S. |
| 6.1 | 8 | 2928 | TOXIC SOLID, CORROSIVE, ORGANIC, N.O.S. |
| 6.1 | 8 | 3289 | TOXIC LIQUID, CORROSIVE, INORGANIC, N.O.S. |
| 6.1 | 8 | 3290 | TOXIC SOLID, CORROSIVE, INORGANIC, N.O.S. |
| 6.1 | 8 | 3389 | TOXIC BY INHALATION LIQUID, CORROSIVE, N.O.S. with an LC50 lower than or equal to 200 mL/m3 and saturated vapour concentration greater than or equal to 500 LC50 |
| 6.1 | 8 | 3390 | TOXIC BY INHALATION LIQUID, CORROSIVE, N.O.S. with an LC50 lower than or equal to 1,000 mL/m3 and saturated vapour concentration greater than or equal to 10 LC50 |

APPENDIX A

| Class or division | Subsidiary hazard | UN No. | Proper shipping name |
| --- | --- | --- | --- |
|  |  |  | CLASS 6 (continued) |
|  |  |  | Class 6.2 |
|  |  |  | Specific entries |
| 6.2 |  | 3291 | CLINICAL WASTE, UNSPECIFIED, N.O.S. or (BIO)MEDICAL WASTE, N.O.S. or REGULATED MEDICAL WASTE, N.O.S. |
| 6.2 |  | 3373 | BIOLOGICAL SUBSTANCE, CATEGORY B |
| 6.2 |  | 3549 | MEDICAL WASTE, CATEGORY A, AFFECTING HUMANS, solid or MEDICAL WASTE, CATEGORY A, AFFECTING ANIMALS only, solid |
|  |  |  | General entries |
| 6.2 |  | 2814 | INFECTIOUS SUBSTANCE, AFFECTING HUMANS |
| 6.2 |  | 2900 | INFECTIOUS SUBSTANCE, AFFECTING ANIMALS only |

APPENDIX A

| Class or division | Subsidiary hazard | UN No. | Proper shipping name |
| --- | --- | --- | --- |
|  |  |  | CLASS 7 |
|  |  |  | General entries |
| 7 |  | 2908 | RADIOACTIVE MATERIAL, EXCEPTED PACKAGE – EMPTY PACKAGING |
| 7 |  | 2909 | RADIOACTIVE MATERIAL, EXCEPTED PACKAGE – ARTICLES MANUFACTURED FROM NATURAL URANIUM or DEPLETED URANIUM or NATURAL THORIUM |
| 7 |  | 2910 | RADIOACTIVE MATERIAL, EXCEPTED PACKAGE – LIMITED QUANTITY OF MATERIAL |
| 7 |  | 2911 | RADIOACTIVE MATERIAL, EXCEPTED PACKAGE – INSTRUMENTS or ARTICLES |
| 7 |  | 2912 | RADIOACTIVE MATERIAL, LOW SPECIFIC ACTIVITY (LSA-I) non fissile or fissile-excepted |
| 7 |  | 2913 | RADIOACTIVE MATERIAL, SURFACE CONTAMINATED OBJECTS (SCO-I, SCO-II or SCO-III) non fissile or fissile-excepted |
| 7 |  | 2915 | RADIOACTIVE MATERIAL, TYPE A PACKAGE, non-special form, non fissile or fissile-excepted |
| 7 |  | 2916 | RADIOACTIVE MATERIAL, TYPE B(U) PACKAGE non fissile or fissile-excepted |
| 7 |  | 2917 | RADIOACTIVE MATERIAL, TYPE B(M) PACKAGE non fissile or fissile-excepted |
| 7 |  | 2919 | RADIOACTIVE MATERIAL, TRANSPORTED UNDER SPECIAL ARRANGEMENT non fissile or fissile-excepted |
| 7 |  | 3321 | RADIOACTIVE MATERIAL, LOW SPECIFIC ACTIVITY (LSA-II), non fissile or fissile-excepted |
| 7 |  | 3322 | RADIOACTIVE MATERIAL, LOW SPECIFIC ACTIVITY (LSA-III), non fissile or fissile-excepted |
| 7 |  | 3323 | RADIOACTIVE MATERIAL, TYPE C PACKAGE non fissile or fissile-excepted |
| 7 |  | 3324 | RADIOACTIVE MATERIAL, LOW SPECIFIC ACTIVITY (LSA-II), FISSILE |
| 7 |  | 3325 | RADIOACTIVE MATERIAL, LOW SPECIFIC ACTIVITY (LSA-III), FISSILE |
| 7 |  | 3326 | RADIOACTIVE MATERIAL, SURFACE CONTAMINATED OBJECTS (SCO-I or SCO-II), FISSILE |
| 7 |  | 3327 | RADIOACTIVE MATERIAL, TYPE A PACKAGE, FISSILE non-special form |
| 7 |  | 3328 | RADIOACTIVE MATERIAL, TYPE B(U) PACKAGE, FISSILE |
| 7 |  | 3329 | RADIOACTIVE MATERIAL, TYPE B(M) PACKAGE, FISSILE |
| 7 |  | 3330 | RADIOACTIVE MATERIAL, TYPE C PACKAGE, FISSILE |
| 7 |  | 3331 | RADIOACTIVE MATERIAL, TRANSPORTED UNDER SPECIAL ARRANGEMENT, FISSILE |
| 7 |  | 3332 | RADIOACTIVE MATERIAL, TYPE A PACKAGE, SPECIAL FORM non fissile or fissile-excepted |
| 7 |  | 3333 | RADIOACTIVE MATERIAL, TYPE A PACKAGE, SPECIAL FORM, FISSILE |

APPENDIX A

| Class or division | Subsidiary hazard | UN No. | Proper shipping name |
| --- | --- | --- | --- |
|  |  |  | CLASS 8 |
|  |  |  | Specific entries |
| 8 |  | 1719 | CAUSTIC ALKALI LIQUID, N.O.S. |
| 8 |  | 1740 | HYDROGENDIFLUORIDES, SOLID, N.O.S. |
| 8 |  | 1903 | DISINFECTANT, LIQUID, CORROSIVE, N.O.S. |
| 8 |  | 2430 | ALKYLPHENOLS, SOLID, N.O.S. (including C2–C12 homologues) |
| 8 |  | 2693 | BISULPHITES, AQUEOUS SOLUTION, N.O.S. |
| 8 |  | 2735 | AMINES, LIQUID, CORROSIVE, N.O.S. or POLYAMINES, LIQUID, CORROSIVE, N.O.S. |
| 8 |  | 2801 | DYE, LIQUID, CORROSIVE, N.O.S. or DYE INTERMEDIATE, LIQUID, CORROSIVE, N.O.S. |
| 8 |  | 2837 | BISULPHATES, AQUEOUS SOLUTION |
| 8 |  | 2987 | CHLOROSILANES, CORROSIVE, N.O.S. |
| 8 |  | 3145 | ALKYLPHENOLS, LIQUID, N.O.S. (including C2–C12 homologues) |
| 8 |  | 3147 | DYE, SOLID, CORROSIVE, N.O.S. or DYE INTERMEDIATE, SOLID, CORROSIVE, N.O.S. |
| 8 |  | 3259 | AMINES, SOLID, CORROSIVE, N.O.S. or POLYAMINES, SOLID, CORROSIVE, N.O.S. |
| 8 | 3 | 2734 | AMINES, LIQUID, CORROSIVE, FLAMMABLE, N.O.S. or POLYAMINES, LIQUID, CORROSIVE, FLAMMABLE, N.O.S. |
| 8 | 3 | 2986 | CHLOROSILANES, CORROSIVE, FLAMMABLE, N.O.S. |
| 8 | 6.1 | 3471 | HYDROGENDIFLUORIDES SOLUTION, N.O.S. |
|  |  |  | General entries |
| 8 |  | 1759 | CORROSIVE SOLID, N.O.S. |
| 8 |  | 1760 | CORROSIVE LIQUID, N.O.S. |
| 8 |  | 3244 | SOLIDS CONTAINING CORROSIVE LIQUID, N.O.S. |
| 8 |  | 3260 | CORROSIVE SOLID, ACIDIC, INORGANIC, N.O.S. |
| 8 |  | 3261 | CORROSIVE SOLID, ACIDIC, ORGANIC, N.O.S. |
| 8 |  | 3262 | CORROSIVE SOLID, BASIC, INORGANIC, N.O.S. |
| 8 |  | 3263 | CORROSIVE SOLID, BASIC, ORGANIC, N.O.S. |
| 8 |  | 3264 | CORROSIVE LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 8 |  | 3265 | CORROSIVE LIQUID, ACIDIC, ORGANIC, N.O.S. |
| 8 |  | 3266 | CORROSIVE LIQUID, BASIC, INORGANIC, N.O.S. |
| 8 |  | 3267 | CORROSIVE LIQUID, BASIC, ORGANIC, N.O.S. |
| 8 | See 2.0.6.6 | 3547 | ARTICLES CONTAINING CORROSIVE SUBSTANCE, N.O.S. |
| 8 | 3 | 2920 | CORROSIVE LIQUID, FLAMMABLE, N.O.S. |
| 8 | 4.1 | 2921 | CORROSIVE SOLID, FLAMMABLE, N.O.S. |
| 8 | 4.2 | 3095 | CORROSIVE SOLID, SELF-HEATING, N.O.S. |
| 8 | 4.2 | 3301 | CORROSIVE LIQUID, SELF-HEATING, N.O.S. |
| 8 | 4.3 | 3094 | CORROSIVE LIQUID, WATER-REACTIVE, N.O.S. |
| 8 | 4.3 | 3096 | CORROSIVE SOLID, WATER-REACTIVE, N.O.S. |
| 8 | 5.1 | 3084 | CORROSIVE SOLID, OXIDIZING, N.O.S. |
| 8 | 5.1 | 3093 | CORROSIVE LIQUID, OXIDIZING, N.O.S. |
| 8 | 6.1 | 2922 | CORROSIVE LIQUID, TOXIC, N.O.S. |
| 8 | 6.1 | 2923 | CORROSIVE SOLID, TOXIC, N.O.S. |

APPENDIX A

| Class or division | Subsidiary hazard | UN No. | Proper shipping name |
| --- | --- | --- | --- |
|  |  |  | CLASS 9 |
|  |  |  | General entries |
| 9 |  | 3077 | ENVIRONMENTALLY HAZARDOUS SUBSTANCE, SOLID, N.O.S. |
| 9 |  | 3082 | ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S. |
| 9 |  | 3245 | GENETICALLY MODIFIED MICROORGANISMS or GENETICALLY MODIFIED ORGANISMS |
| 9 |  | 3257 | ELEVATED TEMPERATURE LIQUID, N.O.S. at or above 100°C and below its flashpoint (including molten metals, molten salts, etc.) |
| 9 |  | 3258 | ELEVATED TEMPERATURE SOLID, N.O.S., at or above 240°C |
| see SP960 |  | 3334 | AVIATION REGULATED LIQUID, N.O.S. |
| see SP960 |  | 3335 | AVIATION REGULATED SOLID, N.O.S. |
| 9 | See 2.0.6.6 | 3548 | ARTICLES CONTAINING MISCELLANEOUS DANGEROUS GOODS, N.O.S. |

APPENDIX A