Chapter 2.10

Marine pollutants

# Definition

Marine pollutants means substances which are subject to the provisions of Annex III of MARPOL, as amended.

# General provisions

      - Marine pollutants shall be transported under the provisions of Annex III of MARPOL, as amended.

      - The Index indicates by the symbol P in the column headed MP those substances, materials and articles that are identified as marine pollutants.

      - Marine pollutants shall be transported under the appropriate entry according to their properties if they fall within the criteria of any of the classes 1 to 8. If they do not fall within the criteria of any of these classes, they shall be transported under the entry: ENVIRONMENTALLY HAZARDOUS SUBSTANCE, SOLID, N.O.S., UN 3077 or ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S., UN 3082, as appropriate,

unless there is a specific entry in class 9.

      - Column 4 of the Dangerous Goods List also provides information on marine pollutants using the symbol P for single entries. The absence of the symbol P or the presence of a “–” in that column does not preclude the application of 2.10.3.

      - When a substance, material or article possesses properties that meet the criteria of a marine pollutant but is not identified in this Code, such substance, material or article shall be transported as a marine pollutant in accordance with the Code.

      - With the approval of the competent authority (see 7.9.2), substances, materials or articles that are identified as marine pollutants in this Code but which no longer meet the criteria as a marine pollutant need not be transported in accordance with the provisions of this Code applicable to marine pollutants.

△ 2.10.2.7  Marine pollutants also meeting the criteria for inclusion in a hazard class from 1 to 9 (except class 7), and packaged in single or combination packagings containing a net quantity per single or inner packaging of 5 L or less for liquids or having a net mass per single or inner packaging of 5 kg or less for solids, are not subject to the provisions of 5.2.1.6, 5.3.2.3, 5.4.1.4.3.6 and 7.1.4.2 of this Code. These provisions do not apply to substances transported under UN 3077 or UN 3082 (see special provision 375).

# Classification

      - Marine pollutants shall be classified in accordance with chapter 2.9.3.

      - The classification criteria of 2.9.3 are not applicable to substances or materials of class 7.