3

MSC 108/20/Add.2

Annex 8, page 569

# Chapter 3.1

General

# Scope and general provisions

      - The Dangerous Goods List in chapter 3.2 lists many of the dangerous goods most commonly transported. The list includes entries for specific chemical substances and articles and generic or “not otherwise specified” entries. Since it is not practical to include a separate entry for every chemical substance or article of commercial importance specifically by name, especially names for mixtures and solutions of various chemical constituents and concentrations, the Dangerous Goods List also includes generic or “not otherwise specified” names (e.g. EXTRACTS, LIQUID, for flavour or aroma, UN 1197 or FLAMMABLE LIQUID, N.O.S., UN 1993). On this basis, the Dangerous Goods List is intended to include an appropriate name or entry for any dangerous good which may be transported.

      - Where a dangerous good is specifically listed by name in the Dangerous Goods List, it shall be transported in accordance with the provisions in the List which are appropriate for that dangerous good. A generic or “not otherwise specified” entry may be used to permit the transport of substances, materials or articles which do not appear specifically by name in the Dangerous Goods List. Such a dangerous good may be transported only after its dangerous properties have been determined. Dangerous goods shall be classified according to the class definitions, tests and criteria. The name which most appropriately describes the dangerous goods shall be used. Only when the specific name of the dangerous goods does not appear in the Dangerous Goods List or the associated primary or subsidiary hazards assigned to it are not appropriate may a generic or “not otherwise specified” name be used. The classification shall be made by the shipper/consignor or by the appropriate competent authority where so specified in the Code. Once the class of the dangerous good has been so established, all conditions for transport, as provided in this Code, shall be met. Any dangerous good having or suspected of having explosive characteristics shall first be considered for inclusion in class 1. Some collective entries may be of the generic or “not otherwise specified” type provided that the Code contains provisions ensuring safety, both by excluding extremely dangerous goods from normal transport and by covering all subsidiary hazards inherent in some goods.

      - Inherent instability in goods may take different dangerous forms, for example explosion, polymerization with intense evolution of heat or emission of flammable, toxic, corrosive or asphyxiant gases. The Dangerous Goods List indicates that certain dangerous goods, or dangerous goods in a specific form, concentration or state, are prohibited for transport by sea. This means that the goods specified are not suitable for transport by sea under normal conditions of transport. This does not mean that such goods may not be transported under any circumstances. For most goods, such inherent instability can be controlled by suitable packaging, dilution, stabilization, addition of an inhibitor, temperature control or other measures.

      - Where precautionary measures are laid down in the Dangerous Goods List in respect of a given dangerous good (such as that it shall be “stabilized” or “with x% water or phlegmatizer”), such dangerous good may not normally be transported when these measures have not been taken, unless the item in question is listed elsewhere (such as class 1) without any indication of, or with different, precautionary measures.

      - Certain substances, by the nature of their chemical composition, tend to polymerize or otherwise react in a dangerous manner under certain conditions of temperature or in contact with a catalyst. Mitigation of this tendency can be carried out either by requiring special transport conditions or by adding adequate amounts of chemical inhibitors or stabilizers to the product. These products shall be sufficiently stabilized to prevent any dangerous reaction during the intended voyage. If this cannot be ensured, the transport of such products is prohibited.

      - Where the contents of a portable tank is to be transported heated, the transport temperature is to be maintained during the intended voyage unless it is established that crystallization or solidification on cooling would not result in instability, which can occur with some stabilized or inhibited products.

3

MSC 108/20/Add.2

Annex 8, page 570

Part 3 – Dangerous Goods List, special provisions and exceptions

# Proper shipping names

Note 1: The proper shipping names of the dangerous goods are those listed in chapter 3.2, Dangerous Goods List. Synonyms, secondary names, initials, abbreviations of names, etc. have been included in the Index to facilitate the search for the proper shipping name (see part 5, Consignment procedures).

Note 2: For proper shipping names to be used for transport of samples, see 2.0.4. For proper shipping names to be used for transport of wastes, see 5.4.1.4.3.3.

      - The proper shipping name is that portion of the entry most accurately describing the goods in the Dangerous Goods List, which is shown in upper-case characters (plus any numbers, Greek letters, ‘sec’, ‘tert’, and the letters m, n, o, p, which form an integral part of the name). An alternative proper shipping name may be shown in brackets following the main proper shipping name (such as ETHANOL (ETHYL ALCOHOL)). Portions of an entry appearing in lower case need not be considered as part of the proper shipping name but may be used.

△ 3.1.2.2	When a combination of several distinct proper shipping names are listed under a single UN number, and these are separated by “or” in lower case or are punctuated by commas, only the most appropriate shall be shown in the transport document and package marks.

Examples illustrating the selection of the proper shipping name for such entries are:

        - UN 1057 LIGHTERS or LIGHTER REFILLS – The proper shipping name is the most appropriate of the following possible combinations:

LIGHTERS LIGHTER REFILLS;

        - UN 2583 ALKYLSULPHONIC ACIDS, SOLID or ARYLSULPHONIC ACIDS, SOLID with more than 5% free

sulphuric acid – The proper shipping name is the most appropriate of the following: ALKYLSULPHONIC ACIDS, SOLID

ARYLSULPHONIC ACIDS, SOLID;

        - UN 2793 FERROUS METAL BORINGS, SHAVINGS, TURNINGS or CUTTINGS in a form liable to

self-heating. The proper shipping name is the most appropriate of the following combinations: FERROUS METAL BORINGS

FERROUS METAL SHAVINGS FERROUS METAL TURNINGS FERROUS METAL CUTTINGS.

      - Proper shipping names may be used in the singular or plural as appropriate. In addition, when qualifying words are used as part of the proper shipping name, their sequence on documentation or packages is optional. Commercial or military names for goods of class 1, which contain the proper shipping name supplemented by additional text, may be used.

      - Many substances have an entry for both the liquid and solid state (see definitions for liquids and solids in 1.2.1), or for the solid and solution. These are allocated separate UN numbers which are not necessarily adjacent to each other. Details are provided in the alphabetical index, e.g.:

NITROXYLENES, LIQUID –	6.1	1665

NITROXYLENES, SOLID  –	6.1	3447.

      - Where it is not already included, the qualifying word “MOLTEN” shall be added to the proper shipping name when a substance which is solid in accordance with the definition in 1.2.1 is offered for transport in the molten state (such as ALKYLPHENOL, SOLID, N.O.S., MOLTEN). For elevated temperature substances, see 5.4.1.4.3.4.

      - Except for self-reactive substances and organic peroxides and unless it is already included in capital letters in the name indicated in the Dangerous Goods List, the word STABILIZED shall be added as part of the proper shipping name of the substance which without stabilization would be forbidden from transport in accordance with 1.1.3 due to it being liable to dangerously react under conditions normally encountered in transport (such as TOXIC LIQUID, ORGANIC, N.O.S., STABILIZED). When temperature control is used to stabilize such substances to prevent the development of any dangerous excess pressure, or the evolution of excessive heat, or when chemical stabilization is used in combination with temperature control, then:

- For liquids and solids where the SAPT (measured without or with inhibitor, when chemical stabilization is applied) is less than or equal to that prescribed in 2.4.2.5.2, special provision 386 of chapter 3.3 and the provisions of 7.3.7 apply;

- Unless it is already included in capital letters in the name indicated in the Dangerous Goods List, the words “TEMPERATURE CONTROLLED” shall be added as part of the proper shipping name;

- For gases: the conditions of transport shall be approved by the competent authority.

      - Hydrates may be transported under the proper shipping name for the anhydrous substance.

3

MSC 108/20/Add.2

Annex 8, page 571

Chapter 3.1 – General

      - Generic or “not otherwise specified” (N.O.S.) entries

        - Generic and “not otherwise specified” proper shipping names that are assigned to special provision 274 or 318 in column 6 of the Dangerous Goods List shall be supplemented with the technical or chemical group names unless a national law or international convention prohibits its disclosure if it is a controlled substance. For explosives of class 1, the dangerous goods description may be supplemented by additional descriptive text to indicate commercial or military names. Technical and chemical group names shall be entered in brackets immediately following the proper shipping name. An appropriate modifier, such as “contains” or “containing” or other qualifying words such as “mixture”, “solution”, etc., and the percentage of the technical constituent may also be used. For example: “UN 1993 Flammable liquid, n.o.s. (contains xylene and benzene), 3, PG II”.

          - The technical name shall be a recognized chemical or biological name or other name currently used in scientific and technical handbooks, journals and texts. Trade names shall not be used for this purpose. In the case of pesticides, only ISO common name(s), other name(s) in The WHO Recommended Classification of Pesticides by Hazard and Guidelines to Classification, or the name(s) of the active substance(s) may be used.

          - When a mixture of dangerous goods or articles containing dangerous goods are described by one of the “N.O.S.” or “generic” entries to which special provision 274 has been allocated in the Dangerous Goods List, not more than the two constituents which most predominantly contribute to the hazard or hazards of the mixture or of the articles need to be shown, excluding controlled substances when their disclosure is prohibited by national law or international convention. If a package containing a mixture is labelled with any subsidiary hazard label, one of the two technical names shown in brackets shall be the name of the constituent which compels the use of the subsidiary hazard label.

          - Examples illustrating the selection of the proper shipping name supplemented with the technical name of goods for such N.O.S. entries are:

UN 2902 PESTICIDE, LIQUID, TOXIC, N.O.S. (drazoxolon)

UN 3394 ORGANOMETALLIC SUBSTANCE, LIQUID, PYROPHORIC, WATER-REACTIVE

(trimethylgallium).

UN 3540 ARTICLES CONTAINING FLAMMABLE LIQUID, N.O.S. (pyrrolidine)

      - Marine pollutants

        - For the purpose of documentation, the proper shipping name of “not otherwise specified” (N.O.S.) entries which are classified as marine pollutants in accordance with 2.10.3, shall be supplemented with the recognized chemical name of the constituent which most predominantly contributes to the classification as marine pollutant unless otherwise provided by SP274.

        - An example illustrating the selection of the proper shipping name supplemented with the recognized technical name of goods for such entries are is indicated below:

UN 1993 FLAMMABLE LIQUID, N.O.S. (propyl acetate, di-n-butyltin di-2-ethylhexanoate) class 3 PG III (50°C c.c.) MARINE POLLUTANT

# Mixtures or solutions

Note: Where a substance is specifically listed by name in the Dangerous Goods List, it shall be identified in transport by the proper shipping name in the Dangerous Goods List. Such substances may contain technical impurities (for example those deriving from the production process) or additives for stability or other purposes that do not affect their classification. However, a substance listed by name containing technical impurities or additives for stability or other purposes affecting its classification shall be considered a mixture or solution (see 2.0.2.2 and 2.0.2.5).

      - A mixture or solution is not subject to the provisions of this Code if the characteristics, properties, form or physical state of the mixture or solution are such that it does not meet the criteria, including human experience criteria, for inclusion in any class.

      - A mixture or solution meeting the classification criteria of this Code composed of a single predominant substance identified by name in the Dangerous Goods List and one or more substances not subject to the provisions of this Code and/or traces of one or more substances identified by name in the Dangerous Goods List, shall be assigned the UN number and proper shipping name of the predominant substance named in the Dangerous Goods List unless:

        - the mixture or solution is identified by name in the Dangerous Goods List;

        - the name and description of the substance named in the Dangerous Goods List specifically indicate that they apply only to the pure substance;

        - the hazard class or division, subsidiary hazard(s), packing group, or physical state of the mixture or solution is different from that of the substance named in the Dangerous Goods List; or

3

MSC 108/20/Add.2

Annex 8, page 572

Part 3 – Dangerous Goods List, special provisions and exceptions

        - the hazard characteristics and properties of the mixture or solution necessitate emergency response measures that are different from those required for the substance identified by name in the Dangerous Goods List.

      - Qualifying words such as “MIXTURE” or “SOLUTION”, as appropriate, shall be added as part of the proper shipping name, for example, “ACETONE SOLUTION”. In addition, the concentration of the mixture or solution may also be indicated after the basic description of the mixture or solution, for example, “ACETONE 75% SOLUTION”.

      - A mixture or solution meeting the classification criteria of this Code that is not identified by name in the Dangerous Goods List and that is composed of two or more dangerous goods shall be assigned to an entry that has the proper shipping name, description, hazard class or division, subsidiary hazard(s) and packing group that most precisely describe the mixture or solution.

# Segregation groups

      - For the purpose of segregation, dangerous goods having certain similar chemical properties have been grouped together in segregation groups, see 7.2.5.

      - It is recognized that not all substances, mixtures, solutions or preparations falling within a segregation group are listed in the IMDG Code by name. These are shipped under N.O.S. entries. Although these N.O.S. entries are not themselves listed in the segregation groups (see 3.1.4.4), the consignor shall decide whether inclusion under the segregation group is appropriate and, if so, shall mention that fact in the transport document (see 5.4.1.5.11).

      - The segregation groups in this Code do not cover substances which fall outside the classification criteria of the Code. It is recognized that some non-hazardous substances have similar chemical properties as substances listed in the segregation groups. A consignor or the person responsible for packing the goods into a cargo transport unit who does have knowledge of the chemical properties of such non-dangerous goods may decide to implement the segregation provisions of a related segregation group on a voluntary basis.

      - The following segregation groups are identified.

- Acids (SGG1)

| 1052 | Hydrogen fluoride, anhydrous |
| --- | --- |
| 1182 | Ethyl chloroformate |
| 1183 | Ethyldichlorosilane |
| 1238 | Methyl chloroformate |
| 1242 | Methyldichlorosilane |
| 1250 | Methyltrichlorosilane |
| 1295 | Trichlorosilane |
| 1298 | Trimethylchlorosilane |
| 1305 | Vinyltrichlorosilane |
| 1572 | Cacodylic acid |
| 1595 | Dimethyl sulphate |
| 1715 | Acetic anhydride |
| 1716 | Acetyl bromide |
| 1717 | Acetyl chloride |
| 1718 | Butyl acid phosphate |
| 1722 | Allyl chloroformate |
| 1723 | Allyl iodide |
| 1724 | Allyltrichlorosilane, stabilized |
| 1725 | Aluminium bromide, anhydrous |
| 1726 | Aluminium chloride, anhydrous |
| 1727 | Ammonium hydrogendifluoride, solid |
| 1728 | Amyltrichlorosilane |
| 1729 | Anisoyl chloride |
| 1730 | Antimony pentachloride, liquid |
| 1731 | Antimony pentachloride solution |

3

|  |  | MSC 108/20/Add.2 Annex 8, page 573 Chapter 3.1 – General |
| --- | --- | --- |
| 1732 | Antimony pentafluoride |  |
| 1733 | Antimony trichloride |  |
| 1736 | Benzoyl chloride |  |
| 1737 | Benzyl bromide |  |
| 1738 | Benzyl chloride |  |
| 1739 | Benzyl chloroformate |  |
| 1740 | Hydrogendifluorides, n.o.s. |  |
| 1742 | Boron trifluoride acetic acid complex, liquid |  |
| 1743 | Boron trifluoride propionic acid complex, liquid |  |
| 1744 | Bromine or bromine solution |  |
| 1745 | Bromine pentafluoride |  |
| 1746 | Bromine trifluoride |  |
| 1747 | Butyltrichlorosilane |  |
| 1750 | Chloroacetic acid solution |  |
| 1751 | Chloroacetic acid, solid |  |
| 1752 | Chloroacetyl chloride |  |
| 1753 | Chlorophenyltrichlorosilane |  |
| 1754 | Chlorosulphonic acid (with or without sulphur trioxide) |  |
| 1755 | Chromic acid solution |  |
| 1756 | Chromic fluoride, solid |  |
| 1757 | Chromic fluoride solution |  |
| 1758 | Chromium oxychloride |  |
| 1762 | Cyclohexenyltrichlorosilane |  |
| 1763 | Cyclohexyltrichlorosilane |  |
| 1764 | Dichloroacetic acid |  |
| 1765 | Dichloroacetyl chloride |  |
| 1766 | Dichlorophenyltrichlorosilane |  |
| 1767 | Diethyldichlorosilane |  |
| 1768 | Difluorophosphoric acid, anhydrous |  |
| 1769 | Diphenyldichlorosilane |  |
| 1770 | Diphenylmethyl bromide |  |
| 1771 | Dodecyltrichlorosilane |  |
| 1773 | Ferric chloride, anhydrous |  |
| 1775 | Fluoroboric acid |  |
| 1776 | Fluorophosphoric acid, anhydrous |  |
| 1777 | Fluorosulphonic acid |  |
| 1778 | Fluorosilicic acid |  |
| 1779 | Formic acid with more than 85% acid by mass |  |
| 1780 | Fumaryl chloride |  |
| 1781 | Hexadecyltrichlorosilane |  |
| 1782 | Hexafluorophosphoric acid |  |
| 1784 | Hexyltrichlorosilane |  |
| 1786 | Hydrofluoric acid and sulphuric acid mixture |  |
| 1787 | Hydriodic acid |  |
| 1788 | Hydrobromic acid |  |
| 1789 | Hydrochloric acid |  |
| 1790 | Hydrofluoric acid |  |
| 1792 | Iodine monochloride, solid |  |
| 1793 | Isopropyl acid phosphate |  |
| 1794 | Lead sulphate with more than 3% free acid |  |

3

MSC 108/20/Add.2

Annex 8, page 574

Part 3 – Dangerous Goods List, special provisions and exceptions

| 1796 | Nitrating acid mixture |
| --- | --- |
| 1798 | Nitrohydrochloric acid |
| 1799 | Nonyltrichlorosilane |
| 1800 | Octadecyltrichlorosilane |
| 1801 | Octyltrichlorosilane |
| 1802 | Perchloric acid with not more than 50% acid, by mass |
| 1803 | Phenolsulphonic acid, liquid |
| 1804 | Phenyltrichlorosilane |
| 1805 | Phosphoric acid, solution |
| 1806 | Phosphorus pentachloride |
| 1807 | Phosphorus pentoxide |
| 1808 | Phosphorus tribromide |
| 1809 | Phosphorus trichloride |
| 1810 | Phosphorus oxychloride |
| 1811 | Potassium hydrogendifluoride, solid |
| 1815 | Propionyl chloride |
| 1816 | Propyltrichlorosilane |
| 1817 | Pyrosulphuryl chloride |
| 1818 | Silicon tetrachloride |
| 1826 | Nitrating acid mixture, spent |
| 1827 | Stannic chloride, anhydrous |
| 1828 | Sulphur chlorides |
| 1829 | Sulphur trioxide, inhibited or sulphur trioxide, stabilized |
| 1830 | Sulphuric acid with more than 51% acid |
| 1831 | Sulphuric acid, fuming |
| 1832 | Sulphuric acid, spent |
| 1833 | Sulphurous acid |
| 1834 | Sulphuryl chloride |
| 1836 | Thionyl chloride |
| 1837 | Thiophosphoryl chloride |
| 1838 | Titanium tetrachloride |
| 1839 | Trichloroacetic acid |
| 1840 | Zinc chloride solution |
| 1848 | Propionic acid with not less than 10% and less than 90% acid, by mass |
| 1873 | Perchloric acid with more than 50% but not more than 72% acid, by mass |
| 1898 | Acetyl iodide |
| 1902 | Diisooctyl acid phosphate |
| 1905 | Selenic acid |
| 1906 | Sludge acid |
| 1938 | Bromoacetic acid solution |
| 1939 | Phosphorus oxybromide |
| 1940 | Thioglycolic acid |
| 2031 | Nitric acid, other than red fuming |
| 2032 | Nitric acid, red fuming |
| 2214 | Phthalic anhydride with more than 0.05% of maleic anhydride |
| 2215 | Maleic anhydride |
| 2218 | Acrylic acid, inhibited |
| 2225 | Benzenesulphonyl chloride |
| 2226 | Benzotrichloride |
| 2240 | Chromosulphuric acid |

3

MSC 108/20/Add.2

Annex 8, page 575

Chapter 3.1 – General

| 2262 | Dimethylcarbamoyl chloride |
| --- | --- |
| 2267 | Dimethyl thiophosphoryl chloride |
| 2305 | Nitrobenzenesulphonic acid |
| 2308 | Nitrosylsulphuric acid, liquid |
| 2331 | Zinc chloride, anhydrous |
| 2353 | Butyryl chloride |
| 2395 | Isobutyryl chloride |
| 2407 | Isopropyl chloroformate |
| 2434 | Dibenzyldichlorosilane |
| 2435 | Ethylphenyldichlorosilane |
| 2437 | Methylphenyldichlorosilane |
| 2438 | Trimethylacetyl chloride |
| 2439 | Sodium hydrogendifluoride |
| 2440 | Stannic chloride pentahydrate |
| 2442 | Trichloroacetyl chloride |
| 2443 | Vanadium oxytrichloride |
| 2444 | Vanadium tetrachloride |
| 2475 | Vanadium trichloride |
| 2495 | Iodine pentafluoride |
| 2496 | Propionic anhydride |
| 2502 | Valeryl chloride |
| 2503 | Zirconium tetrachloride |
| 2506 | Ammonium hydrogen sulphate |
| 2507 | Chloroplatinic acid, solid |
| 2508 | Molybdenum pentachloride |
| 2509 | Potassium hydrogen sulphate |
| 2511 | 2-Chloropropionic acid |
| 2513 | Bromoacetyl bromide |
| 2531 | Methacrylic acid, stabilized |
| 2564 | Trichloroacetic acid solution |
| 2571 | Alkylsulphuric acids |
| 2576 | Phosphorus oxybromide, molten |
| 2577 | Phenylacetyl chloride |
| 2578 | Phosphorus trioxide |
| 2580 | Aluminium bromide solution |
| 2581 | Aluminium chloride solution |
| 2582 | Ferric chloride solution |
| 2583 | Alkylsulphonic acids, solid or arylsulphonic acids, solid with more than 5% free sulphuric acid |
| 2584 | Alkylsulphonic acids, liquid or arylsulphonic acids, liquid with more than 5% free sulphuric acid |
| 2585 | Alkylsulphonic acids, solid or arylsulphonic acids, solid with not more than 5% free sulphuric acid |
| 2586 | Alkylsulphonic acids, liquid or arylsulphonic acids, liquid with not more than 5% free sulphuric acid |
| 2604 | Boron trifluoride diethyl etherate |
| 2626 | Chloric acid, aqueous solution with not more than 10% chloric acid |
| 2642 | Fluoroacetic acid |
| 2670 | Cyanuric chloride |
| 2691 | Phosphorus pentabromide |
| 2692 | Boron tribromide |
| 2698 | Tetrahydrophthalic anhydrides with more than 0.05% maleic anhydride |
| 2699 | Trifluoroacetic acid |

3

MSC 108/20/Add.2

Annex 8, page 576

Part 3 – Dangerous Goods List, special provisions and exceptions

| 2739 | Butyric anhydride |
| --- | --- |
| 2740 | Propyl chloroformate |
| 2742 | Chloroformates, toxic, corrosive, flammable, n.o.s. |
| 2743 | n-Butyl chloroformate |
| 2744 | Cyclobutyl chloroformate |
| 2745 | Chloromethyl chloroformate |
| 2746 | Phenyl chloroformate |
| 2748 | 2-Ethylhexyl chloroformate |
| 2751 | Diethylthiophosphoryl chloride |
| 2789 | Acetic acid, glacial or acetic acid solution, more than 80% acid, by mass |
| 2790 | Acetic acid solution, more than 10% but not more than 80% acid, by mass |
| 2794 | Batteries, wet, filled with acid electric storage |
| 2796 | Sulphuric acid with not more than 51% acid or battery fluid, acid |
| 2798 | Phenylphosphorus dichloride |
| 2799 | Phenylphosphorus thiodichloride |
| 2802 | Copper chloride |
| 2817 | Ammonium hydrogendifluoride solution |
| 2819 | Amyl acid phosphate |
| 2820 | Butyric acid |
| 2823 | Crotonic acid, solid |
| 2826 | Ethyl chlorothioformate |
| 2829 | Caproic acid |
| 2834 | Phosphorous acid |
| 2851 | Boron trifluoride dihydrate |
| 2865 | Hydroxylamine sulphate |
| 2869 | Titanium trichloride mixture |
| 2879 | Selenium oxychloride |
| 2967 | Sulphamic acid |
| 2985 | Chlorosilanes, flammable, corrosive, n.o.s. |
| 2986 | Chlorosilanes, corrosive, flammable, n.o.s. |
| 2987 | Chlorosilanes, corrosive, n.o.s. |
| 2988 | Chlorosilanes, water-reactive, flammable, corrosive, n.o.s. |
| 3246 | Methanesulphonyl chloride |
| 3250 | Chloroacetic acid, molten |
| 3260 | Corrosive solid, acidic, inorganic, n.o.s. |
| 3261 | Corrosive solid, acidic, organic, n.o.s. |
| 3264 | Corrosive liquid, acidic, inorganic, n.o.s. |
| 3265 | Corrosive liquid, acidic, organic, n.o.s. |
| 3277 | Chloroformates, toxic, corrosive, n.o.s. |
| 3361 | Chlorosilanes, toxic, corrosive, n.o.s. |
| 3362 | Chlorosilanes, toxic, corrosive, flammable, n.o.s. |
| 3412 | Formic acid with not less than 10% but not more than 85% acid by mass |
| 3412 | Formic acid with not less than 5% but not more than 10% acid by mass |
| 3419 | Boron trifluoride acetic acid complex, solid |
| 3420 | Boron trifluoride propionic acid complex, solid |
| 3421 | Potassium hydrogendifluoride solution |
| 3425 | Bromoacetic acid, solid |
| 3453 | Phosphoric acid, solid |
| 3456 | Nitrosylsulphuric acid, solid |
| 3463 | Propionic acid with not less than 90% acid by mass |

3

MSC 108/20/Add.2

Annex 8, page 577

Chapter 3.1 – General

3472	Crotonic acid, liquid

3498	Iodine monochloride, liquid

- Ammonium compounds (SGG2)

|  | 0004 | Ammonium picrate dry or wetted with less than 10% water, by mass |
| --- | --- | --- |
|  | 0222 | Ammonium nitrate, with more than 0.2% combustible substances |
|  | 0402 | Ammonium perchlorate |
|  | 1310 | Ammonium picrate, wetted with not less than 10% water, by mass |
|  | 1439 | Ammonium dichromate |
|  | 1442 | Ammonium perchlorate |
|  | 1444 | Ammonium persulphate |
|  | 1546 | Ammonium arsenate |
|  | 1630 | Mercury ammonium chloride |
|  | 1727 | Ammonium hydrogendifluoride, solid |
| ■ | 1835 | Tetramethylammonium hydroxide aqueous solution with more than 2.5% but less than 25% tetramethylammonium hydroxide |
| △ | 1835 | Tetramethylammonium hydroxide aqueous solution with not more than 2.5% tetramethylammonium hydroxide |
|  | 1843 | Ammonium dinitro-o-cresolate, solid |
|  | 1942 | Ammonium nitrate with not more than 0.2% combustible substances |
|  | 2067 | Ammonium nitrate based fertilizer |
|  | 2071 | Ammonium nitrate based fertilizer |
|  | 2073 | Ammonia solution, relative density less than 0.880 at 15°C in water, with more than 35% but not more than 50% ammonia |
|  | 2426 | Ammonium nitrate, liquid (hot concentrated solution) |
|  | 2505 | Ammonium fluoride |
|  | 2506 | Ammonium hydrogen sulphate |
|  | 2683 | Ammonium sulphide solution |
|  | 2687 | Dicyclohexylammonium nitrite |
|  | 2817 | Ammonium hydrogendifluoride solution |
|  | 2818 | Ammonium polysulphide solution |
|  | 2854 | Ammonium fluorosilicate |
|  | 2859 | Ammonium metavanadate |
|  | 2861 | Ammonium polyvanadate |
|  | 2863 | Sodium ammonium vanadate |
|  | 3375 | Ammonium nitrate emulsion or suspension or gel, intermediate for blasting explosives |
|  | 3423 | Tetramethylammonium hydroxide, solid |
|  | 3424 | Ammonium dinitro-o-cresolate solution |
| ■ | 3560 | Tetramethylammonium hydroxide aqueous solution with not less than 25% tetramethylammonium hydroxide |

- Bromates (SGG3)

| 1450 | Bromates, inorganic, n.o.s. |
| --- | --- |
| 1473 | Magnesium bromate |
| 1484 | Potassium bromate |
| 1494 | Sodium bromate |
| 2469 | Zinc bromate |
| 2719 | Barium bromate |
| 3213 | Bromates, inorganic, aqueous solution, n.o.s. |

- Chlorates (SGG4)

1445	Barium chlorate, solid 1452	Calcium chlorate

3

MSC 108/20/Add.2

Annex 8, page 578

Part 3 – Dangerous Goods List, special provisions and exceptions

| 1458 | Chlorate and borate mixture |
| --- | --- |
| 1459 | Chlorate and magnesium chloride mixture, solid |
| 1461 | Chlorates, inorganic, n.o.s. |
| 1485 | Potassium chlorate |
| 1495 | Sodium chlorate |
| 1506 | Strontium chlorate |
| 1513 | Zinc chlorate |
| 2427 | Potassium chlorate, aqueous solution |
| 2428 | Sodium chlorate, aqueous solution |
| 2429 | Calcium chlorate, aqueous solution |
| 2573 | Thallium chlorate |
| 2721 | Copper chlorate |
| 2723 | Magnesium chlorate |
| 3405 | Barium chlorate solution |
| 3407 | Chlorate and magnesium chloride mixture solution |
| Chlorites (SGG5) |  |
| 1453 | Calcium chlorite |
| 1462 | Chlorites, inorganic, n.o.s. |
| 1496 | Sodium chlorite |
| 1908 | Chlorite solution |
| Cyanides (SGG6) |  |
| 1541 | Acetone cyanhydrin, stabilized |
| 1565 | Barium cyanide |
| 1575 | Calcium cyanide |
| 1587 | Copper cyanide |
| 1588 | Cyanides, inorganic, solid, n.o.s. |
| 1620 | Lead cyanide |
| 1626 | Mercuric potassium cyanide |
| 1636 | Mercury cyanide |
| 1642 | Mercury oxycyanide, desensitized |
| 1653 | Nickel cyanide |
| 1679 | Potassium cuprocyanide |
| 1680 | Potassium cyanide, solid |
| 1684 | Silver cyanide |
| 1689 | Sodium cyanide, solid |
| 1694 | Bromobenzyl cyanides, liquid |
| 1713 | Zinc cyanide |
| 1889 | Cyanogen bromide |
| 1935 | Cyanide solution, n.o.s. |
| 2205 | Adiponitrile |
| 2316 | Sodium cuprocyanide, solid |
| 2317 | Sodium cuprocyanide solution |
| 3413 | Potassium cyanide solution |
| 3414 | Sodium cyanide solution |
| 3449 | Bromobenzyl cyanides, solid |
| Heavy metals and their salts (including their organometallic compounds) (SGG7) |  |
| 0129 | Lead azide, wetted, with not less than 20% water, or mixture of alcohol and water, by mass |
| 0130 | Lead styphnate (lead trinitroresorcinate), wetted with not less than 20% water, or mixture of alcohol and water, by mass |

5

6

7

3

MSC 108/20/Add.2

Annex 8, page 579

Chapter 3.1 – General

| 0135 | Mercury fulminate, wetted with not less than 20% water, or mixture of alcohol and water, by |
| --- | --- |
|  | mass |
| 1347 | Silver picrate, wetted with not less than 30% water, by mass |
| 1389 | Alkali metal amalgam, liquid |
| 1392 | Alkaline earth metal amalgam, liquid |
| 1435 | Zinc ashes |
| 1436 | Zinc dust or zinc powder |
| 1469 | Lead nitrate |
| 1470 | Lead perchlorate, solid |
| 1493 | Silver nitrate |
| 1513 | Zinc chlorate |
| 1514 | Zinc nitrate |
| 1515 | Zinc permanganate |
| 1516 | Zinc peroxide |
| 1587 | Copper cyanide |
| 1616 | Lead acetate |
| 1617 | Lead arsenates |
| 1618 | Lead arsenites |
| 1620 | Lead cyanide |
| 1623 | Mercuric arsenate |
| 1624 | Mercuric chloride |
| 1625 | Mercuric nitrate |
| 1626 | Mercuric potassium cyanide |
| 1627 | Mercurous nitrate |
| 1629 | Mercury acetate |
| 1630 | Mercury ammonium chloride |
| 1631 | Mercury benzoate |
| 1634 | Mercury bromides |
| 1636 | Mercury cyanide |
| 1637 | Mercury gluconate |
| 1638 | Mercury iodide |
| 1639 | Mercury nucleate |
| 1640 | Mercury oleate |
| 1641 | Mercury oxide |
| 1642 | Mercury oxycyanide, desensitized |
| 1643 | Mercury potassium iodide |
| 1644 | Mercury salicylate |
| 1645 | Mercury sulphate |
| 1646 | Mercury thiocyanate |
| 1649 | Motor fuel anti-knock mixture |
| 1653 | Nickel cyanide |
| 1674 | Phenylmercuric acetate |
| 1683 | Silver arsenite |
| 1684 | Silver cyanide |
| 1712 | Zinc arsenate and zinc arsenite mixture |
| 1713 | Zinc cyanide |
| 1714 | Zinc phosphide |
| 1794 | Lead sulphate with more than 3% free acid |
| 1838 | Titanium tetrachloride |
| 1840 | Zinc chloride solution |
| 1872 | Lead dioxide |

3

MSC 108/20/Add.2

Annex 8, page 580

Part 3 – Dangerous Goods List, special provisions and exceptions

| 1894 | Phenylmercuric hydroxide |
| --- | --- |
| 1895 | Phenylmercuric nitrate |
| 1931 | Zinc dithionite |
| 1931 | Zinc hydrosulphite |
| 2024 | Mercury compound, liquid, n.o.s. |
| 2025 | Mercury compound, solid, n.o.s. |
| 2026 | Phenylmercuric compound, n.o.s. |
| 2291 | Lead compound, soluble, n.o.s. |
| 2331 | Zinc chloride, anhydrous |
| 2441 | Titanium trichloride, pyrophoric or titanium trichloride mixture, pyrophoric |
| 2469 | Zinc bromate |
| 2546 | Titanium powder, dry |
| 2714 | Zinc resinate |
| 2777 | Mercury based pesticide, solid, toxic |
| 2778 | Mercury based pesticide, liquid, flammable, toxic |
| 2809 | Mercury |
| 2855 | Zinc fluorosilicate |
| 2869 | Titanium trichloride mixture |
| 2878 | Titanium, sponge granules or titanium, sponge powders |
| 2881 | Metal catalyst, dry |
| 2989 | Lead phosphite, dibasic |
| 3011 | Mercury based pesticide, liquid, toxic, flammable |
| 3012 | Mercury based pesticide, liquid, toxic |
| 3089 | Metal powder, flammable, n.o.s. |
| 3174 | Titanium disulphide |
| 3181 | Metal salts of organic compounds, flammable, n.o.s. |
| 3189 | Metal powder, self-heating, n.o.s. |
| 3401 | Alkali metal amalgam, solid |
| 3402 | Alkaline earth metal amalgam, solid |
| 3408 | Lead perchlorate solution |
| 3483 | Motor fuel anti-knock mixture, flammable |
| Hypochlorites (SGG8) |  |
| 1471 | Lithium hypochlorite |
| 1748 | Calcium hypochlorite, dry or calcium hypochlorite mixture, dry with more than 39% available chlorine (8.8% available oxygen) |
| 1791 | Hypochlorite solution |
| 2208 | Calcium hypochlorite mixture, dry with more than 10% but not more than 39% available chlorine |
| 2741 | Barium hypochlorite with more than 22% available chlorine |
| 2880 | Calcium hypochlorite, hydrated or calcium hypochlorite, hydrated mixture with not less than 5.5% but not more than 16% water |
| 3212 | Hypochlorites, inorganic, n.o.s. |
| 3255 | tert-Butyl hypochlorite |
| 3485 | Calcium hypochlorite, dry, corrosive or calcium hypochlorite mixture, dry, corrosive with more than 39% available chlorine (8.8% available oxygen) |
| 3486 | Calcium hypochlorite mixture, dry, corrosive with more than 10% but not more than 39% available chlorine |
| 3487 | Calcium hypochlorite, hydrated, corrosive or calcium hypochlorite, hydrated mixture, corrosive, with not less than 5.5% but not more than 16% water |

8

9	Lead and its compounds (SGG9)

0129	Lead azide, wetted with not less than 20% water, or mixture of alcohol and water, by mass

3

MSC 108/20/Add.2

Annex 8, page 581

Chapter 3.1 – General

| 0130 | Lead styphnate, wetted with not less than 20% water, or mixture of alcohol and water, by mass |
| --- | --- |
| 0130 | Lead trinitroresorcinate, wetted with not less than 20% water, or mixture of alcohol and water, by mass |
| 1469 | Lead nitrate |
| 1470 | Lead perchlorate, solid |
| 1616 | Lead acetate |
| 1617 | Lead arsenates |
| 1618 | Lead arsenites |
| 1620 | Lead cyanide |
| 1649 | Motor fuel anti-knock mixture |
| 1794 | Lead sulphate with more than 3% free acid |
| 1872 | Lead dioxide |
| 2291 | Lead compound, soluble, n.o.s. |
| 2989 | Lead phosphide, dibasic |
| 3408 | Lead perchlorate solution |
| 3483 | Motor fuel anti-knock mixture, flammable |
| Liquid halogenated hydrocarbons (SGG10) |  |
| 1099 | Allyl bromide |
| 1100 | Allyl chloride |
| 1107 | Amyl chloride |
| 1126 | 1-Bromobutane |
| 1127 | Chlorobutanes |
| 1134 | Chlorobenzene |
| 1150 | 1,2-Dichloroethylene |
| 1152 | Dichloropentanes |
| 1184 | Ethylene dichloride |
| 1278 | 1-Chloropropane |
| 1279 | 1,2-Dichloropropane |
| 1303 | Vinylidene chloride, stabilized |
| 1591 | o-Dichlorobenzene |
| 1593 | Dichloromethane |
| 1605 | Ethylene dibromide |
| 1647 | Methyl bromide and ethylene dibromide mixture, liquid |
| 1669 | Pentachloroethane |
| 1701 | Xylyl bromide |
| 1702 | 1,1,2,2-Tetrachloroethane |
| 1710 | Trichloroethylene |
| 1723 | Allyl iodide |
| 1737 | Benzyl bromide |
| 1738 | Benzyl chloride |
| 1846 | Carbon tetrachloride |
| 1887 | Bromochloromethane |
| 1888 | Chloroform |
| 1891 | Ethyl bromide |
| 1897 | Tetrachloroethylene |
| 1991 | Chloroprene, stabilized |
| 2234 | Chlorobenzotrifluorides |
| 2238 | Chlorotoluenes |
| 2279 | Hexachlorobutadiene |
| 2321 | Trichlorobenzenes, liquid |

10

3

MSC 108/20/Add.2

Annex 8, page 582

Part 3 – Dangerous Goods List, special provisions and exceptions

2322	Trichlorobutene

2339	2-Bromobutane

2341	1-Bromo-3-methylbutane

2342	Bromomethylpropanes

2343	2-Bromopentane

2344	Bromopropanes

2356	2-Chloropropane

2362	1,1-Dichloroethane

2387	Fluorobenzene

2388	Fluorotoluenes

2390	2-Iodobutane

2391	Iodomethylpropanes

2392	Iodopropanes

2456	2-Chloropropene

2504	Tetrabromoethane

2515	Bromoform

2554	Methylallyl chloride

2644	Methyl iodide

2646	Hexachlorocyclopentadiene

2664	Dibromomethane

2688	1-Bromo-3-chloropropane

2831	1,1,1-Trichloroethane

2872	Dibromochloropropanes

11  Mercury and mercury compounds (SGG11)

0135	Mercury fulminate, wetted with not less than 20% water, or mixture of alcohol and water, by mass

1389	Alkali metal amalgam, liquid

1392	Alkaline earth metal amalgam, liquid 1623	Mercuric arsenate

1624	Mercuric chloride

1625	Mercuric nitrate

1626	Mercuric potassium cyanide 1627	Mercurous nitrate

1629	Mercury acetate

1630	Mercury ammonium chloride 1631	Mercury benzoate

1634	Mercury bromides

1636	Mercury cyanide

1637	Mercury gluconate

1638	Mercury iodide

1639	Mercury nucleate

1640	Mercury oleate

1641	Mercury oxide

1642	Mercury oxycyanide, desensitized 1643	Mercury potassium iodide

1644	Mercury salicylate

1645	Mercury sulphate

1646	Mercury thiocyanate

1894	Phenylmercuric hydroxide

1895	Phenylmercuric nitrate

3

|  |  | MSC 108/20/Add.2 Annex 8, page 583 Chapter 3.1 – General |
| --- | --- | --- |
| 2024 | Mercury compound, liquid, n.o.s. |  |
| 2025 | Mercury compound, solid, n.o.s. |  |
| 2026 | Phenylmercuric compound, n.o.s. |  |
| 2777 | Mercury based pesticide, solid, toxic |  |
| 2778 | Mercury based pesticide, liquid, flammable, toxic |  |
| 2809 | Mercury |  |
| 3011 | Mercury based pesticide, liquid, toxic, flammable |  |
| 3012 | Mercury based pesticide, liquid, toxic |  |
| 3401 | Alkali metal amalgam, solid |  |
| 3402 | Alkaline earth metal amalgam, solid |  |
| 12 Nitrites and their mixtures (SGG12) |  |  |
| 1487 | Potassium nitrate and sodium nitrite mixture |  |
| 1488 | Potassium nitrite |  |
| 1500 | Sodium nitrite |  |
| 2627 | Nitrites, inorganic, n.o.s. |  |
| 2726 | Nickel nitrite |  |
| 3219 | Nitrites, inorganic, aqueous solution, n.o.s. |  |
| 13 Perchlorates (SGG13) |  |  |
| 1442 | Ammonium perchlorate |  |
| 1447 | Barium perchlorate, solid |  |
| 1455 | Calcium perchlorate |  |
| 1470 | Lead perchlorate, solid |  |
| 1475 | Magnesium perchlorate |  |
| 1481 | Perchlorates, inorganic, n.o.s. |  |
| 1489 | Potassium perchlorate |  |
| 1502 | Sodium perchlorate |  |
| 1508 | Strontium perchlorate |  |
| 3211 | Perchlorates, inorganic, aqueous solution, n.o.s. |  |
| 3406 | Barium perchlorate solution |  |
| 3408 | Lead perchlorate solution |  |
| 14 Permanganates (SGG14) |  |  |
| 1448 | Barium permanganate |  |
| 1456 | Calcium permanganate |  |
| 1482 | Permanganates, inorganic, n.o.s. |  |
| 1490 | Potassium permanganate |  |
| 1503 | Sodium permanganate |  |
| 1515 | Zinc permanganate |  |
| 3214 | Permanganates, inorganic, aqueous solution, n.o.s. |  |
| 15 Powdered metals (SGG15) |  |  |
| 1309 | Aluminium powder, coated |  |
| 1326 | Hafnium powder, wetted with not less than 25% water |  |
| 1352 | Titanium powder, wetted with not less than 25% water |  |
| 1358 | Zirconium powder, wetted with not less than 25% water |  |
| 1383 | Pyrophoric alloy or pyrophoric metal, n.o.s. |  |
| 1396 | Aluminium powder, uncoated |  |
| 1398 | Aluminium silicon powder, uncoated |  |
| 1418 | Magnesium powder or magnesium alloys powder |  |
| 1435 | Zinc ashes |  |
| 1436 | Zinc dust or zinc powder |  |
| 1854 | Barium alloys, pyrophoric |  |

3

MSC 108/20/Add.2

Annex 8, page 584

Part 3 – Dangerous Goods List, special provisions and exceptions

| 2008 | Zirconium powder, dry |
| --- | --- |
| 2009 | Zirconium, dry, sheets, strip or coiled wire |
| 2545 | Hafnium powder, dry |
| 2546 | Titanium powder, dry |
| 2878 | Titanium sponge powders |
| 2881 | Metal catalyst, dry |
| 2950 | Magnesium granules, coated, particle size not less than 149 microns |
| 3078 | Cerium, turnings or gritty powder |
| 3089 | Metal powder, flammable, n.o.s. |
| 3170 | Aluminium smelting by-products or aluminium remelting by-products |
| 3189 | Metal powder, self-heating, n.o.s. |
| Peroxides (SGG16) |  |
| 1449 | Barium peroxide |
| 1457 | Calcium peroxide |
| 1472 | Lithium peroxide |
| 1476 | Magnesium peroxide |
| 1483 | Peroxides, inorganic, n.o.s. |
| 1491 | Potassium peroxide |
| 1504 | Sodium peroxide |
| 1509 | Strontium peroxide |
| 1516 | Zinc peroxide |
| 2014 | Hydrogen peroxide, aqueous solution, 20–60% |
| 2015 | Hydrogen peroxide, aqueous solution, stabilized |
| 2466 | Potassium superoxide |
| 2547 | Sodium superoxide |
| 3149 | Hydrogen peroxide and peroxyacetic acid mixture |
| 3377 | Sodium perborate monohydrate |
| 3378 | Sodium carbonate peroxyhydrate |
| Azides (SGG17) |  |
| 0129 | Lead azide, wetted with not less than 20% water, or mixture of alcohol and water, by mass |
| 0224 | Barium azide, dry or wetted with less than 50% water, by mass |
| 1571 | Barium azide, wetted with not less than 50% water, by mass |
| 1687 | Sodium azide |
| Alkalis (SGG18) |  |
| 1005 | Ammonia, anhydrous |
| 1160 | Dimethylamine, aqueous solution |
| 1163 | Dimethylhydrazine, unsymmetrical |
| 1235 | Methylamine, aqueous solution |
| 1244 | Methylhydrazine |
| 1289 | Sodium methylate solution in alcohol |
| 1382 | Potassium sulphide, anhydrous or potassium sulphide with less than 30% water of crystallization |
| 1385 | Sodium sulphide, anhydrous or sodium sulphide with less than 30% water of crystallization |
| 1431 | Sodium methylate |
| 1604 | Ethylenediamine |
| 1719 | Caustic alkali liquid, n.o.s. |
| 1813 | Potassium hydroxide, solid |
| 1814 | Potassium hydroxide solution |
| 1819 | Sodium aluminate solution |
| 1823 | Sodium hydroxide, solid |
| 1824 | Sodium hydroxide solution |

16

17

18

3

MSC 108/20/Add.2

Annex 8, page 585

Chapter 3.1 – General

|  | 1825 | Sodium monoxide |
| --- | --- | --- |
| ■ | 1835 | Tetramethylammonium hydroxide aqueous solution with more than 2.5% but less than 25% tetramethylammonium hydroxide |
| △ | 1835 | Tetramethylammonium hydroxide aqueous solution with not more than 2.5% tetramethylammonium hydroxide |
|  | 1847 | Potassium sulphide, hydrated with not less than 30% water of crystallization |
|  | 1849 | Sodium sulphide, hydrated with not less than 30% water |
|  | 1907 | Soda lime with more than 4% sodium hydroxide |
|  | 1922 | Pyrrolidine |
|  | 2029 | Hydrazine, anhydrous |
|  | 2030 | Hydrazine, aqueous solution with more than 37% hydrazine, by mass |
|  | 2033 | Potassium monoxide |
|  | 2073 | Ammonia solution, relative density less than 0.880 at 15°C in water, with more than 35% but not more than 50% ammonia |
|  | 2079 | Diethylenetriamine |
|  | 2259 | Triethylenetetramine |
|  | 2270 | Ethylamine, aqueous solution, with not less than 50% but not more than 70% ethylamine |
|  | 2318 | Sodium hydrosulphide with less than 25% water of crystallization |
|  | 2320 | Tetraethylenepentamine |
|  | 2379 | 1,3-Dimethylbutylamine |
|  | 2382 | Dimethylhydrazine, symmetrical |
|  | 2386 | 1-Ethylpiperidine |
|  | 2399 | 1-Methylpiperidine |
|  | 2401 | Piperidine |
|  | 2491 | Ethanolamine or ethanolamine solution |
|  | 2579 | Piperazine |
|  | 2671 | Aminopyridines (o-, m-, p-) |
|  | 2672 | Ammonia solution relative density between 0.880 and 0.957 at 15°C in water, with more than 10% but not more than 35% ammonia, by mass |
|  | 2677 | Rubidium hydroxide solution |
|  | 2678 | Rubidium hydroxide |
|  | 2679 | Lithium hydroxide solution |
|  | 2680 | Lithium hydroxide |
|  | 2681 | Caesium hydroxide solution |
|  | 2682 | Caesium hydroxide |
|  | 2683 | Ammonium sulphide solution |
|  | 2733 | Amines, flammable, corrosive, n.o.s. or polyamines, flammable, corrosive, n.o.s. |
|  | 2734 | Amines, liquid, corrosive, flammable, n.o.s. or polyamines, liquid, corrosive, flammable, n.o.s. |
|  | 2735 | Amines, liquid, corrosive, n.o.s. or polyamines, liquid, corrosive, n.o.s. |
|  | 2795 | Batteries, wet, filled with alkali, electric storage |
|  | 2797 | Battery fluid, alkali |
|  | 2818 | Ammonium polysulphide solution |
|  | 2949 | Sodium hydrosulphide, hydrated with not less than 25% water of crystallization |
|  | 3028 | Batteries, dry, containing potassium hydroxide, solid electric storage |
|  | 3073 | Vinylpyridines, stabilized |
|  | 3206 | Alkali metal alcoholates, self-heating, corrosive, n.o.s. |
|  | 3253 | Disodium trioxosilicate |
|  | 3259 | Amines, solid, corrosive, n.o.s. or polyamines, solid, corrosive, n.o.s. |
|  | 3262 | Corrosive solid, basic, inorganic, n.o.s. |
|  | 3263 | Corrosive solid, basic, organic, n.o.s. |
|  | 3266 | Corrosive liquid, basic, inorganic, n.o.s. |

3

MSC 108/20/Add.2

Annex 8, page 586

Part 3 – Dangerous Goods List, special provisions and exceptions

3267	Corrosive liquid, basic, organic, n.o.s. 3274	Alcoholates solution, n.o.s. in alcohol

3293	Hydrazine, aqueous solution with not more than 37% hydrazine, by mass

3318	Ammonia solution, relative density less than 0.880 at 15°C in water, with more than 50% ammonia

3320	Sodium borohydride and sodium hydroxide solution with not more than 12% sodium borohydride and not more than 40% sodium hydroxide, by mass

3423	Tetramethylammonium hydroxide, solid

3484	Hydrazine aqueous solution, flammable, with more than 37% hydrazine, by mass

- 3560	Tetramethylammonium hydroxide aqueous solution with not less than 25% tetramethylammonium hydroxide