Chapter 7.4

Stowage and segregation on containerships

Note:	To facilitate familiarization with these requirements and to support training of relevant personnel, illustrations applicable to the segregation requirements on containerships are given in MSC.1/Circ.1440.

# Introduction

      - The provisions of this chapter apply to the stowage and segregation of containers which meet the definition of a container within the terms of the International Convention for Safe Containers, 1972 (CSC Convention), as amended, which are transported on deck and in the cargo holds of containerships or on deck and in the cargo holds of other types of ships provided that these stowage positions are properly fitted to give a permanent stowage of containers during transport.

      - For ships carrying containers in conventional cargo spaces not properly fitted for the permanent stowage of containers the provisions of chapter 7.6 apply.

      - For stowage of FISH MEAL, UNSTABILIZED (UN 1374), FISH MEAL, STABILIZED (UN 2216) and KRILL MEAL

(UN 3497) in containers, the provisions of 7.6.2.7.2.2 also apply.

      - For stowage of AMMONIUM NITRATE (UN 1942), AMMONIUM NITRATE BASED FERTILIZER (UN 2067 AND

2071) in containers, the applicable provisions of 7.6.2.8.4 and 7.6.2.11.1 also apply.

# Stowage requirements

## Provisions for hatchless containerships

Dangerous goods shall only be transported in or vertically above hatchless container holds if:

- the dangerous goods are permitted for under deck stowage as specified in the Dangerous Goods List; and

- the hatchless container hold is in full compliance with the provisions of regulation II-2/19 of SOLAS, as amended, or regulation II-2/54 of SOLAS, as amended by the resolutions indicated in II-2/1.2.1, as applicable.

## Provisions for ships with partially weathertight hatchway covers

### Provisions for partially weathertight hatchway covers with effective gutterbars*

          - Partially weathertight hatchway covers fitted with effective gutterbars* can be regarded as “resistant to fire and liquid” for the purpose of stowage and segregation of containers containing dangerous goods on containerships fitted with such hatchway covers. Additionally segregation requirements shall be in accordance with the requirements in paragraph 7.4.3.2.

          - When “not in the same vertical line unless separated by a deck” is required, containers containing dangerous goods shall not be stowed in any tier directly above a clear gap* unless the cargo hold complies with the relevant requirements for the class and flashpoint of the dangerous goods in regulation II-2/19 of SOLAS 74, as amended, or regulation II-2/54 of SOLAS, as amended by resolutions indicated in II-2/1.2.1, as applicable. Additionally, containers containing incompatible dangerous goods shall not be stowed within the relevant sensitive vertical lines* under deck.

### Provisions for partially weathertight hatchway covers without effective gutterbars*

7

Where hatchway covers are not fitted with effective gutterbars, containers containing dangerous goods shall not be stowed on such hatchway covers, unless the cargo hold complies with the relevant requirements

* For definitions and details see MSC/Circ.1087 found in the IMDG Code Supplement.

502	IMDG Code (Amendment 42-24) 2024 EDITION

Chapter 7.4 – Stowage and segregation on containerships

for the class and flashpoint of the dangerous goods in regulation II-2/19 of SOLAS 74, as amended, or regulation II-2/54 of SOLAS, as amended by the resolutions indicated in II-2/1.2.1, as applicable.

          - Where hatchway covers are not fitted with effective gutterbars,* the following applies where stowage “not in the same vertical line” is required in 7.4.3.3.

          - When containers containing dangerous goods are stowed on deck, containers containing incompatible dangerous goods shall not be stowed within the relevant sensitive vertical lines* of any clear gap* on either side of the hatchway cover below deck.

          - When containers containing dangerous goods are stowed below deck within the relevant sensitive vertical lines of a clear gap, containers with incompatible dangerous goods shall not be stowed on the hatches above the hold.*

## Provisions for containers with flammable gases and highly flammable liquids

        - In cargo ships of 500 gross tons or over and passenger ships constructed before 1 September 1984, and in cargo ships of less than 500 gross tons constructed before 1 February 1992, containers with flammable gases or with flammable liquids having a flashpoint of less than 23°C c.c, shall be stowed on deck only, unless otherwise approved by the Administration.

        - A container with flammable gases or flammable liquids having a flashpoint of less than 23°C c.c transported on deck shall be stowed at least 2.4 m horizontally and projected vertically away from any potential source of ignition.

        - A container under temperature control that is not of a certified safe type shall not be stowed under deck together with containers containing flammable gases or with liquids having a flashpoint of less than 23°C c.c.

## Ventilation provisions

        - In cargo ships of 500 gross tons or over and passenger ships constructed before 1 September 1984, and in cargo ships of less than 500 gross tons constructed before 1 February 1992, containers with the following dangerous goods may be stowed under deck only if the cargo space is equipped with mechanical ventilation and if under deck stowage is permitted in the Dangerous Goods List:

          - dangerous goods of class 2.1;

          - dangerous goods of class 3 with a flashpoint of less than 23°C c.c.;

          - dangerous goods of class 4.3;

          - dangerous goods of class 6.1 with a subsidiary hazard of class 3;

          - dangerous goods of class 8 with a subsidiary hazard of class 3; and

          - dangerous goods to which a specific stowage requirement requiring mechanical ventilation in column 16a of the Dangerous Goods List is assigned.

Otherwise containers shall be stowed on deck only.

        - The capacity of the mechanical ventilation (number of air changes per hour) shall be to the satisfaction of the Administration.

# Segregation requirements

## Definition and application

        - Container space means a distance of not less than 6 m fore and aft or not less than 2.4 m athwartships.

7

The provisions for segregation between containers on board containerships with closed cargo holds and on board hatchless containerships are given in the tables in 7.4.3.2 and 7.4.3.3, respectively.

* For definitions and details see MSC/Circ.1087 found in the IMDG Code Supplement.

IMDG Code (Amendment 42-24) 2024 EDITION	503

MSC 108/20/Add.2

Annex 8, page 515

Part 7 – Provisions concerning transport operations

7

504

IMDG Code (Amendment 42-24) 2024 EDITION

| Segregation requirement | Vertical |  |  | Horizontal |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Closed versus closed | Closed versus open | Open versus open |  | Closed versus closed |  | Closed versus open |  | Open versus open |  |
|  |  |  |  |  | On deck | Under deck | On deck | Under deck | On deck | Under deck |
| “Away from” | One on top of the | Open on top of closed permitted | Not in the same vertical line unless segregated by a deck | Fore and aft | No restriction | No restriction | No restriction | No restriction | One container space | One container space or one bulkhead |
| .1 | other permitted | Otherwise as for “open versus open” |  | Athwartships | No restriction | No restriction | No restriction | No restriction | One container space | One container space |
| “Separated from” | Not in the same vertical line unless segregated by a deck |  |  | Fore and aft | One container space | One container space or one bulkhead | One container space | One container space or one bulkhead | One container space | One bulkhead |
|  |  |  |  | Athwartships |  |  |  |  |  | One bulkhead |
| .2 |  | As for “open versus |  |  | One container space | One container space | One container space | Two container spaces | Two container spaces |  |
|  |  |  |  | Fore and aft |  | One bulkhead |  | One bulkhead |  | Two bulkheads |
| “Separated by |  | open” |  |  | One container |  | One container |  | Two container |  |
| a complete |  |  |  |  | space |  | space |  | spaces |  |
| compartment |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Athwartships |  | One bulkhead |  | One bulkhead |  | Two bulkheads |
| or hold from” |  |  |  |  | Two container |  | Two container |  | Three container |  |
| .3 |  |  |  |  | spaces |  | spaces |  | spaces |  |
| “Separated longitudinally by an intervening complete compartment or hold from” .4* | Prohibited |  |  | Fore and aft | Minimum horizontal distance of 24 m | One bulkhead and minimum horizontal distance of 24 m* | Minimum horizontal distance of 24 m | Two bulkheads | Minimum horizontal distance of 24 m | Two bulkheads |
|  |  |  |  | Athwartships | Prohibited | Prohibited | Prohibited | Prohibited | Prohibited | Prohibited |

* Containers not less than 6 m from intervening bulkhead.

Note: All bulkheads and decks shall be resistant to fire and liquids.

MSC 108/20/Add.2

Annex 8, page 516

Chapter 7.4 – Stowage and segregation on containerships

7

IMDG Code (Amendment 42-24) 2024 EDITION

505

| Segregation requirement | Vertical |  |  | Horizontal |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Closed versus closed | Closed versus open | Open versus open |  | Closed versus closed |  | Closed versus open |  | Open versus open |  |
|  |  |  |  |  | On deck | Under deck | On deck | Under deck | On deck | Under deck |
| “Away from” .1 | One on top of the other permitted | Open on top of closed permitted otherwise as for “open versus open” | Not in the same vertical line | Fore and aft | No restriction | No restriction | No restriction | No restriction | One container space | One container space or one bulkhead |
|  |  |  |  | Athwartships | No restriction | No restriction | No restriction | No restriction | One container space | One container space |
| “Separated from” .2 | Not in the same vertical line | As for “open versus open” |  | Fore and aft | One container space | One container space or one bulkhead | One container space | One container space or one bulkhead | One container space and not in or above same hold | One bulkhead |
|  |  |  |  | Athwartships | One container space | One container space | Two container spaces | Two container spaces | Two container spaces and not in or above same hold | One bulkhead |
| “Separated by a complete compartment or hold from” .3 |  |  |  | Fore and aft | One container space and not in or above same hold | One bulkhead | One container space and not in or above same hold | One bulkhead | Two container spaces and not in or above same hold | Two bulkheads |
|  |  |  |  | Athwartships | Two container spaces and not in or above same hold | One bulkhead | Two container spaces and not in or above same hold | One bulkhead | Three container spaces and not in or above same hold | Two bulkheads |
| “Separated longitudinally by an intervening complete compartment or hold from” .4 | Prohibited |  |  | Fore and aft | Minimum horizontal distance of 24 m and not in or above same hold | One bulkhead and minimum horizontal distance of 24 m* | Minimum horizontal distance of 24 m and not in or above same hold | Two bulkheads | Minimum horizontal distance of 24 m and not in or above same hold | Two bulkheads |
|  |  |  |  | Athwartships | Prohibited | Prohibited | Prohibited | Prohibited | Prohibited | Prohibited |

* Containers not less than 6 m from intervening bulkhead.

Note: All bulkheads and decks shall be resistant to fire and liquids.